# GoBall - System Architecture Document

Mini Golf Scoring System for Raspberry Pi 5

Ahmed Abdelghani

February 2026

Table of Contents

# 1	Introduction

## 1.1	Purpose

GoBall is a Raspberry Pi 5-based mini golf scoring system that combines hardware sensors, LED feedback, audio announcements, and a touchscreen UI into an interactive scoring experience. This document describes the system architecture, module interactions, hardware interfaces, and data flow.

## 1.2	Scope

This document covers:

- Hardware components and wiring

- Software module architecture

- Game mode logic and state management

- Sensor input pipeline

- Audio and LED subsystems

- UI framework and screen management

- Build and deployment infrastructure

## 1.3	Definitions

| Term | Definition |
| --- | --- |
| LVGL | Light and Versatile Graphics Library - embedded UI framework |
| PIO | Programmable I/O - RPi5 hardware for driving LED protocols |
| WS2812 | Addressable RGB LED protocol (NeoPixel) |
| libgpiod | Linux GPIO character device library |
| SDL2 | Simple DirectMedia Layer - display/audio backend |
| SquareLine Studio | Visual LVGL UI design tool |

# 2	System Overview

## 2.1	High-Level Architecture

+----------------------------------------------------------+
|                    Raspberry Pi 5                         |
|                                                          |
|  +------------+    +-----------+    +----------------+   |
|  | Touchscreen|<-->|   SDL2    |<-->|     LVGL       |   |
|  | (Display)  |    | (Backend) |    | (UI Framework) |   |
|  +------------+    +-----------+    +-------+--------+   |
|                                             |            |
|                         +-------------------+            |
|                         |                                |
|               +---------v----------+                     |
|               |    Main Loop       |                     |
|               |  (5ms tick cycle)  |                     |
|               +-+-------+-------+-+                     |
|                 |       |       |                         |
|        +--------+  +----+----+ +--------+                |
|        |           |         |          |                |
|  +-----v-----+ +--v------+ +v-------+ +v---------+     |
|  |  GPIO/IR  | |  Game   | | Sound  | |   LED    |     |
|  |  Sensors  | |  Logic  | | System | |  System  |     |
|  +-----+-----+ +--+------+ +---+----+ +----+-----+     |
|        |           |            |           |            |
+--------|-----------|------------|-----------|------------+
         |           |            |           |
   +-----v---+  +---v----+  +---v-----+ +---v--------+
   | 4x IR   |  | Player |  | Speaker | | WS2812 LED |
   | Sensors |  | State  |  | + Amp   | | Strips x2  |
   +---------+  +--------+  +---------+ +------------+

## 2.2	Component Summary

| Component | Technology | Purpose |
| --- | --- | --- |
| Display | 7” touchscreen via SDL2/DRM | User interface rendering |
| UI Framework | LVGL v9 + SquareLine Studio | Screen layout, widgets, events |
| Game Logic | Custom C modules | Scoring rules, turn management |
| Sensors | libgpiod + GPIO interrupts | Ball detection (4 scoring holes) |
| Audio | SDL2_mixer | Voice announcements, sound effects |
| LEDs | PIO + WS2812 protocol | Color-coded scoring feedback |
| Build | CMake + aarch64 cross-compiler | Cross-compilation for RPi5 |

# 3	Hardware Architecture

## 3.1	Block Diagram

+---------------------+
                    |   Raspberry Pi 5    |
                    |                     |
   IR Sensors       |  GPIO Header        |    LED Strips
   +---------+      |  +-----------+      |    +----------+
   | 3pt (H1)|----->|--| GPIO 17   |      |    | Strip 1  |
   +---------+      |  +-----------+      |    | (30 LEDs)|
   | 4pt (H2)|----->|--| GPIO 26   |      |    +----+-----+
   +---------+      |  +-----------+      |         |
   | 5pt (H3)|----->|--| GPIO 27   |      |  +------+
   +---------+      |  +-----------+      |  |
   | 0pt (H4)|----->|--| GPIO 24   |      |  | PIO
   +---------+      |  +-----------+      |  |
                    |                     |  +------+
                    |  +-----------+      |         |
                    |  | GPIO 3    |------|--->+----+-----+
                    |  +-----------+      |    | Strip 2  |
                    |  | GPIO 2    |------|--->| (30 LEDs)|
                    |  +-----------+      |    +----------+
                    |                     |
                    |  +-----------+      |    +----------+
                    |  | HDMI/DSI  |------|--->| 7" Touch |
                    |  +-----------+      |    | Display  |
                    |                     |    +----------+
                    |  +-----------+      |    +----------+
                    |  | Audio Out |------|--->| Speaker  |
                    |  +-----------+      |    | + Amp    |
                    +---------------------+    +----------+

## 3.2	GPIO Pin Assignment

| GPIO Pin | Function | Signal Type | Direction |
| --- | --- | --- | --- |
| 17 | 3-Point Hole Sensor | Falling edge interrupt | Input |
| 26 | 4-Point Hole Sensor | Falling edge interrupt | Input |
| 27 | 5-Point Hole Sensor | Falling edge interrupt | Input |
| 24 | 0-Point Hole Sensor | Falling edge interrupt | Input |
| 3 | LED Strip 1 Data | PIO-driven WS2812 | Output |
| 2 | LED Strip 2 Data | PIO-driven WS2812 | Output |

## 3.3	IR Sensor Specifications

- Type: Active IR break-beam sensors

- Detection: Falling edge (beam broken = ball passed)

- Debounce: 3000ms signal-based POSIX timer per sensor

- Interface: /dev/gpiochip0 via libgpiod

## 3.4	LED Strip Specifications

- Type: WS2812 (NeoPixel) addressable RGB+W

- Pixel Count: 30 per strip (configurable via PIXELS define)

- Data Rate: 800 kHz

- Interface: PIO (Programmable I/O) hardware on RPi5

- Color Order: WBGR (White, Blue, Green, Red)

- Brightness: Software-controllable 0-255

# 4	Software Architecture

## 4.1	Module Structure

SquareLine_Project/
|
+-- main.c                          Entry point, main loop
|
+-- modules/
|   +-- debug/                      Debug logging subsystem
|   |   +-- debug.h/c               Configurable per-module logging
|   |
|   +-- game_modes/                 Game rule implementations
|   |   +-- game_modes.h            GameMode, HoleMode, MatchPlayMode enums
|   |   +-- player.h                Player struct definition
|   |   +-- strokeplay.h/c          Stroke Play scoring
|   |   +-- matchplay.h/c           Match Play scoring
|   |   +-- quotaplay.h/c           Quota + Vegas Quota scoring
|   |
|   +-- logic/                      Core game flow controller
|   |   +-- gpio_event.h/c          GPIO init, event loop, turn management,
|   |                                scorecard updates, player highlights
|   |
|   +-- sound_logic/                Audio subsystem
|   |   +-- sound_logic_event.h/c   SDL2_mixer init, WAV loading,
|   |                                timed playback, mute control
|   |
|   +-- led_logic/                  LED strip subsystem
|   |   +-- led_logic_event.h/c     PIO init, animations, flash effects
|   |
|   +-- ui_logic/                   UI event handlers
|       +-- ui_logic.event.h/c      Button callbacks, screen transitions,
|                                    game mode setup from UI selections
|
+-- ui/                             SquareLine Studio generated UI
|   +-- ui.h/c                      UI globals, screen init, event wiring
|   +-- screens/                    All screen definitions
|   +-- images/                     Image assets (C arrays)
|   +-- fonts/                      Font assets (C arrays)
|   +-- components/                 Reusable UI components
|
+-- utils/
|   +-- piolib/                     PIO library for WS2812 control
|   +-- autostart/                  Kiosk mode desktop files
|
+-- lvgl/                           LVGL library source (v9)

## 4.2	Module Dependency Graph

+----------+
                    |  main.c  |
                    +----+-----+
                         |
          +--------------+----------------+
          |              |                |
    +-----v------+ +----v-----+  +-------v--------+
    | gpio_event | | led_logic |  | sound_logic    |
    | (logic/)   | |           |  |                |
    +--+----+----+ +-----+----+  +-------+--------+
       |    |            |               |
  +----+  +-+---+   +---+----+    +-----+------+
  |       |     |   | piolib |    | SDL2_mixer |
  v       v     v   +--------+    +------------+
game_   ui_    ui/
modes/  logic/  (LVGL
               screens)

## 4.3	Main Loop

The application runs a continuous loop at approximately 200 Hz (5ms sleep):

while (1)
{
    update_led_animation(&leds);        // Step 1: Update LED train animation
    logic_handle_events(...);           // Step 2: Poll GPIO, process scoring
    lv_timer_handler();                 // Step 3: LVGL tick (UI render + input)
    usleep(5000);                       // Step 4: 5ms sleep
}

Step 1 advances the LED train animation by one pixel position each frame.

Step 2 performs a non-blocking poll of GPIO lines. If a sensor fires, the full scoring chain executes synchronously: debounce check, scoring function call, detection count increment, UI label update, completion check, and turn advancement.

Step 3 runs LVGL’s timer handler, which processes pending UI events, renders dirty regions, and handles SDL input (touch, mouse).

# 5	Sensor Input Pipeline

## 5.1	Event Flow

IR Beam Broken
      |
      v
GPIO Falling Edge (via libgpiod)
      |
      v
gpiod_line_event_wait_bulk()  [non-blocking]
      |
      v
sensors_enabled check  ----NO----> Ignore
      |
     YES
      v
pin_to_sensor_index(pin_offset)
      |
      v
Debounce Check (3s timer)  ----ACTIVE----> Ignore
      |
    CLEAR
      v
start_debounce_timer()
      |
      v
Game Mode Switch
      |
      +-> Stroke Play:  stroke_play_process_pin()
      +-> Match Play:   inline scoring (3/4/5/0 pts)
      +-> Quota:        quota_play_process_pin()
      +-> Vegas:        vegas_quota_play_process_pin()
      |
      v
detection_count++
      |
      v
logic_update_label_text()  [update ball counter UI]
      |
      v
Quota/Vegas: Check completion (par3==0 && par4==0 && par5==0)
      |
      v
detection_count == SENSORS_PER_TURN (2)?
      |
     YES
      v
Turn Completion Logic
      |
      +-> Stroke Play: update_scoreCard(), advance hole
      +-> Match Play:  evaluate hole winner, update Up&Down
      +-> Quota/Vegas: advance hole, announce next player
      |
      v
current_player_index = (current_player_index + 1) % num_players
      |
      v
check_all_players_completed()  [end-of-game detection]

## 5.2	Debounce Mechanism

Each sensor has an independent POSIX real-time timer using SIGRTMIN signals:

- On first trigger: sensor_debouncing[index] = true, timer starts (3000ms)

- During debounce window: all subsequent triggers on that sensor are ignored

- On timer expiry: signal handler sets sensor_debouncing[index] = false

- Each sensor debounces independently (different holes can trigger simultaneously)

## 5.3	Turn Structure

Each player gets 2 ball detections per turn (SENSORS_PER_TURN = 2). After 2 detections:

- Scores are finalized for the turn

- Scorecard is updated

- Player index advances to the next player

- Player turn voice announcement plays

Exception: Quota/Vegas 1-player mode uses 1 detection per turn.

# 6	Game Modes

## 6.1	Stroke Play

| Parameter | Value |
| --- | --- |
| Players | 1-4 |
| Holes | 9 or 18 |
| Scoring | Cumulative points per hole |
| Win Condition | Highest total score after all holes |

Scoring: Each ball detection adds points based on which hole it enters (3, 4, 5, or 0 points). Two detections per turn are summed as the hole score.

Turn Cycling:

- 2 players: alternates every 8 detections (4 turns each)

- 3 players: cycles every 12 detections

- 4 players: cycles every 24 detections

## 6.2	Match Play

| Parameter | Value |
| --- | --- |
| Mode | 1v1 or 2v2 |
| Holes | 9 or 18 |
| Scoring | Up & Down per hole |
| Win Condition | Lead exceeds remaining holes (early victory) or most holes won |

Scoring: Both players/teams play the same hole. The higher scorer wins the hole. Scores are displayed as “XUP” / “XDN” / “E” (Even).

Early Victory: If a player’s lead exceeds the number of remaining holes, the match ends immediately.

## 6.3	Quota Points

| Parameter | Value |
| --- | --- |
| Players | 1-4 |
| Holes | 9 or 18 |
| Scoring | Par countdown (3pt, 4pt, 5pt targets) |
| Win Condition | First to complete all three par categories (reach 0,0,0) |

Scoring: Each player starts with preset par targets (e.g., par3=3, par4=2, par5=1). Hitting a hole decrements the corresponding counter. The game ends when a player reaches 0 in all three categories.

## 6.4	Vegas Quota Points

| Parameter | Value |
| --- | --- |
| Players | 1-4 |
| Holes | 9 or 18 |
| Scoring | Par countdown + bonus points |
| Win Condition | First to complete; ties broken by bonus score |

Scoring: Same countdown as Quota Points, but after a player completes a category (reaches 0), subsequent hits in that category earn bonus points.

Dead Category Rule: In multiplayer (2+), a category becomes “dead” (no bonus scoring) once 2 players have completed it. This prevents runaway bonus accumulation.

Category Lifecycle:
  OPEN ──(player completes)──> OPEN (1 closed)
       ──(2nd player completes)──> DEAD (no more bonus for anyone)

# 7	Audio Subsystem

## 7.1	Architecture

+------------------+     +----------------+     +----------+
| Sound Logic      |     | SDL2_mixer     |     | Speaker  |
| (sound_logic.c)  |---->| (Mix_Chunk)    |---->| + Amp    |
+------------------+     +----------------+     +----------+
        |
        v
  +------------+
  | WAV Files  |
  | (game_     |
  |  sounds/)  |
  +------------+

## 7.2	Sound Categories

| Category | Examples | Trigger |
| --- | --- | --- |
| Scoring | 3points.wav, 4points.wav, 5points.wav, 0points.wav | Ball detection |
| Player Turn | playerone.wav through playerfour.wav | Turn switch |
| Team Turn | teamone.wav, teamtwo.wav | Match Play turn switch |
| Winner | p1wins1-5.wav, t1wins1-5.wav | Game completion |
| Navigation | mainmenu.wav, goingback.wav | Button press |

## 7.3	Timed Playback

Sound playback uses LVGL timers for non-blocking operation:

play_sound_once(Mix_Chunk* sound, uint32_t delay_ms);

This creates a one-shot LVGL timer that plays the sound after the specified delay. This prevents audio from blocking the main loop and allows staggering of sequential announcements.

## 7.4	Configurable Delays

| Define | Default | Purpose |
| --- | --- | --- |
| SOUND_DELAY_PLAYER_ANNOUNCE_MS | 750ms | Player turn announcement |
| SOUND_DELAY_TEAM_ANNOUNCE_MS | 750ms | Team turn announcement |
| SOUND_DELAY_PLAYER_WINS_MS | 750ms | Winner announcement |
| SOUND_DELAY_TURN_SWITCH_MS | 500ms | Turn switch announcement |

# 8	LED Subsystem

## 8.1	Architecture

+------------------+     +----------+     +----------+     +----------+
| LED Logic        |     | PIO      |     | WS2812   |     | LED      |
| (led_logic.c)    |---->| Hardware |---->| Protocol |---->| Strips   |
+------------------+     +----------+     +----------+     +----------+
        |
        +-- Animation engine (train pattern)
        +-- Flash controller (scoring feedback)
        +-- Brightness control (0-255)

## 8.2	Color Format

Colors use WBGR order (White, Blue, Green, Red) as wbgr_color_t:

| Constant | W | B | G | R | Visual |
| --- | --- | --- | --- | --- | --- |
| COLOR_RED | 0x00 | 0x00 | 0x00 | 0xFF | Red |
| COLOR_GREEN | 0x00 | 0x00 | 0xFF | 0x00 | Green |
| COLOR_BLUE | 0x00 | 0xFF | 0x00 | 0x00 | Blue |
| COLOR_WHITE | 0xFF | 0xFF | 0xFF | 0xFF | White |

## 8.3	LED Behaviors

Idle Animation: A “train” pattern of colored pixels travels along the strip. Multiple trains (defined by NUM_TRAINS) are evenly spaced and advance one pixel per main loop iteration.

Scoring Flash: On ball detection, the animation pauses and LEDs flash:

- Points scored (3/4/5): Green flash for 1000ms

- Zero points: White flash for 1000ms

- Flash toggles on/off at 50ms intervals

- Animation automatically resumes after flash duration

## 8.4	Graceful Degradation

The LED subsystem initializes with graceful fallback:

- pio_open(0) - attempt to open PIO hardware

- If PIO unavailable (e.g., desktop/QEMU): controller.enabled = false

- If state machines unavailable: controller.enabled = false

- All LED functions check controller->enabled before hardware access

# 9	UI Architecture

## 9.1	Framework Stack

+----------------------------------+
| SquareLine Studio (Design Tool)  |
| Exports C code for screens,     |
| widgets, and event callbacks     |
+----------------+-----------------+
                 |
                 v
+----------------------------------+
| LVGL v9 (Runtime)               |
| Handles rendering, input,       |
| animations, and event dispatch   |
+----------------+-----------------+
                 |
                 v
+----------------------------------+
| SDL2 Backend                     |
| Window management, mouse/touch,  |
| keyboard input                   |
+----------------------------------+

## 9.2	Screen Hierarchy

The UI contains screens for each game mode variant:

Main Menu
|
+-- Stroke Play
|   +-- Player Select (1P/2P/3P/4P)
|   +-- Hole Select (9H/18H)
|   +-- Game Screen (per variant: SP1P9H, SP2P18H, etc.)
|   +-- Scorecard Screen
|
+-- Match Play
|   +-- Mode Select (1v1/2v2)
|   +-- Hole Select (9H/18H)
|   +-- Game Screen (MP1V19H, MP2V218H, etc.)
|   +-- Scorecard Screen
|
+-- Quota Points
|   +-- Player Select (1P/2P/3P/4P)
|   +-- Hole Select (9H/18H)
|   +-- Game Screen (Q1P9H, Q4P18H, etc.)
|
+-- Vegas Quota Points
    +-- Player Select (1P/2P/3P/4P)
    +-- Hole Select (9H/18H)
    +-- Game Screen (VQ1P9H, VQ4P18H, etc.)

## 9.3	UI Widget Naming Convention

SquareLine Studio generates widget names following this pattern:

ui_[Mode][Players][Holes]GS[Widget][Detail]

Examples:

| Widget Name | Meaning |
| --- | --- |
| ui_SP2P9HGSBCPText | Stroke Play, 2-Player, 9-Hole, Game Screen, Ball Counter, Player Text |
| ui_MP1V19HScP1SText3 | Match Play, 1v1, 9-Hole, Scorecard, Player 1, Score Text, Hole 3 |
| ui_VQ3P18HGSBSText | Vegas Quota, 3-Player, 18-Hole, Game Screen, Bonus Score Text |
| ui_Q4P9HGSMMButton | Quota, 4-Player, 9-Hole, Game Screen, Main Menu Button |

## 9.4	Event Handling

UI events are wired in two layers:

- SquareLine-generated callbacks (ui/ui.c): Button presses trigger navigation and sound macros

- Custom logic callbacks (modules/ui_logic/ui_logic.event.c): Game setup, score display, player highlighting, screen transitions on game completion

# 10	State Management

## 10.1	Global Game State

| Variable | Type | Purpose |
| --- | --- | --- |
| current_game_mode | GameMode | Active game mode enum |
| current_hole_mode | HoleMode | 9 or 18 holes |
| current_match_play_mode | MatchPlayMode | 1v1 or 2v2 |
| players[] | Player[4] | Player state array |
| current_player_index | int | Whose turn it is |
| num_players | int | Active player count |
| sensors_enabled | bool | Sensor input gate |
| update_flag | uint8_t | Game completion flag |

## 10.2	Player Struct

typedef struct {
    uint8_t score;              // Cumulative score (Stroke/Quota)
    uint8_t current_hole;       // Current hole number
    uint8_t detection_count;    // Detections in current turn
    uint8_t round_total_score;  // Per-round score (Match Play)
    int8_t  upAndDown;          // Match Play standing
    uint8_t holes_won;          // Match Play holes won
    uint8_t par3_count;         // Quota: remaining 3pt pars
    uint8_t par4_count;         // Quota: remaining 4pt pars
    uint8_t par5_count;         // Quota: remaining 5pt pars
} Player;

## 10.3	State Transitions

UI Button Press
                          |
                          v
                   set_game_mode()
                   set_hole_mode()
                   set_num_players()
                   reset_scores()
                          |
                          v
              +--- GAME ACTIVE ---+
              |                   |
         Sensor Event        Game Complete
              |                   |
              v                   v
        Score Update        check_all_players_completed()
        Turn Advance              |
              |                   v
              +-------<---  Show Winner / Scorecard
                            sensors_enabled = false

# 11	Debug System

## 11.1	Architecture

The debug system provides configurable per-module logging with 5 severity levels:

| Level | Value | Macro | Use Case |
| --- | --- | --- | --- |
| ERROR | 1 | DEBUG_ERROR() | Fatal/unrecoverable issues |
| WARN | 2 | DEBUG_WARN() | Unexpected but handled conditions |
| INFO | 3 | DEBUG_INFO() | Significant operational events |
| DEBUG | 4 | DEBUG_DEBUG() | Detailed diagnostic information |
| TRACE | 5 | DEBUG_TRACE() | Function entry/exit, per-frame data |

## 11.2	Build-Type Mapping

| Build Type | Debug Level | Defines |
| --- | --- | --- |
| Debug | 4 (DEBUG) | DEBUG=1 DEBUG_LEVEL=4 |
| Debug + Trace | 5 (TRACE) | DEBUG=1 DEBUG_LEVEL=5 ENABLE_DEBUG_TRACE |
| Release | 2 (WARN) | DEBUG_LEVEL=2 |

## 11.3	Module Tags

Each log line is tagged with a module identifier for filtering:

MODULE_MAIN, MODULE_LVGL, MODULE_GPIO, MODULE_GAME, MODULE_SOUND, MODULE_LED, MODULE_UI, MODULE_HAL, MODULE_LOGIC

## 11.4	Features

- Colored output: ANSI color codes per severity level (configurable via ENABLE_DEBUG_COLORS)

- Timestamps: Millisecond-precision timestamps (configurable via ENABLE_DEBUG_TIMESTAMP)

- Zero overhead in Release: Macros compile to no-ops when level is below threshold

# 12	Build and Deployment

## 12.1	Build Pipeline

+------------------+     +-------------------+     +------------------+
| Host Machine     |     | Cross-Compilation |     | Raspberry Pi 5   |
| (x86_64 Linux)   |     |                   |     | (aarch64)        |
|                  |     | aarch64-linux-    |     |                  |
| Source Code      |---->| gnu-gcc           |---->| Deploy via SCP   |
| CMakeLists.txt   |     | + rpi5-sysroot/   |     | Run on target    |
| toolchain-       |     |                   |     |                  |
|   aarch64.cmake  |     | Output: ELF       |     |                  |
+------------------+     | aarch64 binary    |     +------------------+
                         +-------------------+

## 12.2	CMake Configuration

# Standard build
cmake -B build -DCMAKE_EXPORT_COMPILE_COMMANDS=ON
make -C build -j$(nproc)

# Release build
cmake -B build -DCMAKE_BUILD_TYPE=Release

# With trace logging
cmake -B build -DENABLE_DEBUG_TRACE=ON

# Custom sysroot
cmake -B build -DSYSROOT_PATH=/path/to/rpi5-sysroot

## 12.3	Build Options

| Option | Default | Description |
| --- | --- | --- |
| CMAKE_BUILD_TYPE | Debug | Debug (level 4) or Release (level 2) |
| ENABLE_DEBUG_TRACE | OFF | Enable TRACE-level logging |
| ENABLE_DEBUG_COLORS | ON | Colored terminal output |
| ENABLE_DEBUG_TIMESTAMP | ON | Timestamps in log output |
| SYSROOT_PATH | rpi5-sysroot/ | RPi5 filesystem sysroot |
| SOUND_DIR_PATH | – | Override sound file directory |
| YOCTO_BUILD | – | Skip local toolchain paths |

## 12.4	Dependencies

### 12.4.1	Host (Build Machine)

| Package | Purpose |
| --- | --- |
| cmake >= 3.15 | Build system |
| aarch64-linux-gnu-gcc | Cross-compiler |
| pkg-config | Library discovery |
| rpi5-sysroot/ | Target filesystem headers/libs |

### 12.4.2	Target (Raspberry Pi 5)

| Package | Purpose |
| --- | --- |
| libsdl2-2.0-0 | Display backend |
| libsdl2-mixer-2.0-0 | Audio playback |
| libgpiod2 | GPIO access |

## 12.5	Deployment

# Copy binary to Pi
scp build/SquareLine_Project q@<rpi-ip>:~/Desktop/SquareLine_Project/

# Run on Pi
ssh q@<rpi-ip> './Desktop/SquareLine_Project/SquareLine_Project'

# 13	Appendix A: File Reference

| File | Lines | Purpose |
| --- | --- | --- |
| main.c | ~175 | Application entry, main loop, HAL init |
| modules/logic/gpio_event.c | ~4100 | GPIO init, scoring chain, turn management, scorecard |
| modules/logic/gpio_event.h | ~97 | Pin definitions, game state externs, function prototypes |
| modules/ui_logic/ui_logic.event.c | ~6500 | UI callbacks, score reset, scorecard updates, highlights |
| modules/game_modes/strokeplay.c | ~50 | Stroke play per-pin scoring |
| modules/game_modes/matchplay.c | ~50 | Match play per-pin scoring |
| modules/game_modes/quotaplay.c | ~195 | Quota + Vegas quota scoring with dead-category rule |
| modules/game_modes/player.h | ~30 | Player struct definition |
| modules/game_modes/game_modes.h | ~30 | GameMode, HoleMode, MatchPlayMode enums |
| modules/sound_logic/sound_logic_event.c | ~200 | Audio init, WAV loading, timed playback |
| modules/led_logic/led_logic_event.c | ~315 | PIO init, LED animation, flash effects |
| modules/debug/debug.c | ~100 | Debug init, level config, formatted output |
| ui/ui.c | ~7400 | SquareLine-generated screens and event wiring |

# 14	Appendix B: Enum Definitions

typedef enum {
    GAME_MODE_STROKE_PLAY,
    GAME_MODE_MATCH_PLAY,
    GAME_MODE_QUOTA,
    GAME_MODE_VEGAS
} GameMode;

typedef enum {
    NINE_HOLES = 9,
    EIGHTEEN_HOLES = 18
} HoleMode;

typedef enum {
    MATCH_PLAY_MODE_1V1,
    MATCH_PLAY_MODE_2V2
} MatchPlayMode;