# GoBall - System Design Document

Mini Golf Scoring System for Raspberry Pi 5

Ahmed Abdelghani

February 2026

Table of Contents

# 1	Introduction

## 1.1	Purpose

This System Design Document (SDD) describes the detailed design of the GoBall mini golf scoring system. It covers data structures, algorithms, interface contracts, error handling strategies, and design decisions for each software module. This document complements the System Architecture Document (SAD) which provides the high-level structural overview.

## 1.2	Scope

GoBall is a Raspberry Pi 5-based scoring system that integrates:

- 4 IR sensors for ball detection across scoring holes (3, 4, 5, and 0 points)

- WS2812 LED strips for visual scoring feedback

- SDL2-based touchscreen UI built with LVGL and SquareLine Studio

- Voice announcements via SDL2_mixer

- Support for 5 game modes: Stroke Play, Match Play 1v1, Match Play 2v2, Quota Points, and Vegas Quota Points

## 1.3	Applicable Documents

| Document | Description |
| --- | --- |
| System Architecture Document | High-level module structure and data flow |
| Software Requirements Specification | Functional and non-functional requirements |
| README.md | Build instructions and changelog |

## 1.4	Design Methodology

The system follows an event-driven, single-threaded architecture with a cooperative main loop. Design priorities in order:

- Reliability — hardware failures must not crash the system

- Responsiveness — UI must remain fluid during sensor processing

- Simplicity — flat module structure, minimal abstraction layers

- Testability — graceful degradation enables desktop testing via QEMU

# 2	Design Constraints

## 2.1	Hardware Constraints

| Constraint | Value | Impact |
| --- | --- | --- |
| Target CPU | ARM Cortex-A76 (RPi5, aarch64) | Cross-compilation required |
| GPIO interface | /dev/gpiochip0 via libgpiod | Kernel character device API |
| LED protocol | WS2812 at 800 kHz | Requires PIO hardware; unavailable on x86 |
| Display | 2560x720 touchscreen | Wide dual-panel layout for game + scorecard |
| Audio output | 3.5mm analog or HDMI | SDL2_mixer with WAV files |

## 2.2	Software Constraints

| Constraint | Value | Impact |
| --- | --- | --- |
| Language | C11 | No C++ features; compatible with embedded toolchains |
| UI framework | LVGL v9.1.0 | Screen definitions exported from SquareLine Studio |
| Display backend | SDL2 | Single-window rendering with direct mode |
| Color depth | 32-bit XRGB8888 | Set in lv_conf.h as LV_COLOR_DEPTH = 32 |
| Memory budget | 512 MB LVGL heap | LV_MEM_SIZE = 512 * 1024 * 1024 |
| Refresh rate | 62.5 Hz | LV_DEF_REFR_PERIOD = 16 ms |
| Build system | CMake >= 3.15 | Cross-toolchain via toolchain-aarch64.cmake |

## 2.3	Design Decisions

| Decision | Rationale |
| --- | --- |
| Single-threaded main loop | Avoids mutex complexity; LVGL is not thread-safe |
| Inline scoring in gpio_event.c | All game state mutations in one file prevents race conditions |
| POSIX signal-based debounce | More reliable than timestamp comparison; per-sensor independence |
| WAV files (not MP3/OGG) | Zero decoding latency; SDL2_mixer plays directly |
| PIO for LEDs (not bit-bang) | Precise 800 kHz timing without CPU blocking |
| Static player array | Player players[4] avoids heap allocation for game state |

# 3	Data Design

## 3.1	Player Data Structure

typedef struct Player {
    int         score;              // Cumulative score (Stroke Play, Quota)
    int         current_hole;       // Zero-based hole index (0..8 or 0..17)
    int         detection_count;    // Detections in current turn (0..2)
    int         holes_won;          // Match Play: holes won count
    char        match_status[10];   // Match Play: status string
    int         holes_halved;       // Match Play: halved holes count
    signed char upAndDown;          // Match Play: differential (-N..+N)
    int         round_total_score;  // Per-round cumulative (Match Play)
    int         par3_count;         // Quota: remaining 3pt pars
    int         par4_count;         // Quota: remaining 4pt pars
    int         par5_count;         // Quota: remaining 5pt pars
} Player;

Storage: Static array Player players[MAX_PLAYERS] where MAX_PLAYERS = 4.

Lifecycle:

- Zeroed on logic_initialize_game() call

- Par counts set by UI callbacks when entering Quota/Vegas game screens

- Modified by scoring functions during gameplay

- Reset by reset_scores() on game exit or new game

Field usage by game mode:

| Field | Stroke Play | Match Play | Quota | Vegas |
| --- | --- | --- | --- | --- |
| score | Cumulative | – | Bonus score | Bonus score |
| current_hole | Yes | Yes | Yes | Yes |
| detection_count | Yes | Yes | Yes | Yes |
| holes_won | – | Yes | – | – |
| upAndDown | – | Yes | – | – |
| round_total_score | – | Per-hole | Per-hole | Per-hole |
| par3_count | – | – | Countdown | Countdown |
| par4_count | – | – | Countdown | Countdown |
| par5_count | – | – | Countdown | Countdown |

## 3.2	LED Controller Data Structure

#define NUM_TRAINS    5
#define TRAIN_LENGTH  10
#define PIXELS        144

typedef struct {
    PIO     pio;                            // PIO hardware handle
    uint8_t sm1, sm2;                       // State machine IDs
    uint8_t offset;                         // Program offset in PIO memory
    uint8_t gpio1, gpio2;                   // Data pins (default: 3, 2)
    uint8_t train_positions[NUM_TRAINS];    // Animation positions
    uint8_t databuf1[PIXELS * 4];           // Strip 1: 576 bytes (WBGR)
    uint8_t databuf2[PIXELS * 4];           // Strip 2: 576 bytes (WBGR)
    uint8_t brightness;                     // Global brightness (0-255)
    bool    enabled;                        // Hardware availability flag
} led_strip_controller_t;

Memory layout per pixel (4 bytes):

| Byte offset | Channel | Bit position |
| --- | --- | --- |
| 4*i + 0 | White | MSB |
| 4*i + 1 | Blue |  |
| 4*i + 2 | Green |  |
| 4*i + 3 | Red | LSB |

Global instance: led_strip_controller_t leds (single global, not heap-allocated).

## 3.3	Color Constants

typedef struct {
    uint8_t w, b, g, r;
} wbgr_color_t;

const wbgr_color_t COLOR_RED   = {0x00, 0x00, 0x00, 0xFF};
const wbgr_color_t COLOR_GREEN = {0x00, 0x00, 0xFF, 0x00};
const wbgr_color_t COLOR_BLUE  = {0x00, 0xFF, 0x00, 0x00};
const wbgr_color_t COLOR_WHITE = {0xFF, 0xFF, 0xFF, 0xFF};

## 3.4	Flash Context

typedef struct {
    led_strip_controller_t* controller;
    wbgr_color_t            color;
} flash_context_t;

Heap-allocated via malloc() in trigger_flash_with_color(). Ownership passes to the LVGL timer system; freed implicitly when the timer completes.

## 3.5	Game State Enumerations

typedef enum {
    GAME_MODE_STROKE_PLAY,    // 0
    GAME_MODE_MATCH_PLAY,     // 1
    GAME_MODE_QUOTA,          // 2
    GAME_MODE_VEGAS,          // 3
    NUM_GAME_MODES            // 4 (sentinel)
} GameMode;

#define NINE_HOLES     8      // Zero-based max index for 9 holes
#define EIGHTEEN_HOLES 17     // Zero-based max index for 18 holes

typedef enum {
    HOLES_9  = NINE_HOLES,    // 8
    HOLES_18 = EIGHTEEN_HOLES // 17
} HoleMode;

typedef enum {
    MATCH_PLAY_MODE_1V1,      // 0
    MATCH_PLAY_MODE_2V2       // 1
} MatchPlayMode;

Design note: NINE_HOLES = 8 and EIGHTEEN_HOLES = 17 are zero-based maximum indices, not hole counts. A player has completed all holes when current_hole > NINE_HOLES (i.e., current_hole >= 9).

## 3.6	Global Game State Variables

| Variable | Type | Defined In | Purpose |
| --- | --- | --- | --- |
| current_game_mode | GameMode | gpio_event.c | Active game mode |
| current_hole_mode | HoleMode | gpio_event.c | 9 or 18 holes |
| current_match_play_mode | MatchPlayMode | gpio_event.c | 1v1 or 2v2 |
| players[MAX_PLAYERS] | Player[4] | gpio_event.c | Player state |
| current_player_index | int | gpio_event.c | Active player (0-based) |
| num_players | int | gpio_event.c | Active player count |
| sensors_enabled | bool | gpio_event.c | Sensor input gate |
| update_flag | uint8_t | gpio_event.c | Game completion flag |
| sensor_debouncing[4] | bool[4] | gpio_event.c | Per-sensor debounce state |
| leds | led_strip_controller_t | led_logic_event.c | LED controller instance |

# 4	Module Detailed Design

## 4.1	GPIO and Game Logic Module (modules/logic/gpio_event.c)

### 4.1.1	Responsibilities

- GPIO hardware initialization via libgpiod

- Sensor event polling (non-blocking)

- Debounce management (POSIX signal timers)

- Scoring dispatch to game mode handlers

- Turn management and player rotation

- Scorecard UI updates

- Game completion detection

### 4.1.2	Initialization Sequence

logic_initialize_game(num_players)
    |
    +-> Validate num_players (1..MAX_PLAYERS)
    +-> Zero-initialize players[0..num_players-1]
    +-> init_debounce_timers()
    |       +-> Create 4 POSIX timers (one per sensor)
    |       +-> Register SIGRTMIN signal handler
    |       +-> Handler clears sensor_debouncing[index] on expiry
    +-> logic_gpio_init()
            +-> gpiod_chip_open("/dev/gpiochip0")
            +-> gpiod_chip_get_lines(pins[17,26,27,24])
            +-> gpiod_line_request_bulk_falling_edge_events()

Error handling: Each step returns -1 on failure. main.c catches failure and sets gpio_available = 0, enabling UI-only mode.

### 4.1.3	Debounce Algorithm

Sensor Event Arrives (GPIO falling edge)
    |
    +-> sensor_debouncing[sensor_index] == true?
    |       YES -> Ignore event (timer still active)
    |       NO  -> Continue processing
    |
    +-> sensor_debouncing[sensor_index] = true
    +-> arm_timer(sensor_index, DEBOUNCE_TIME_MS=3000)
    +-> Process scoring event
    |
    ... 3 seconds later ...
    |
    SIGRTMIN fires -> debounce_signal_handler()
    +-> sensor_debouncing[sensor_index] = false
    +-> Sensor ready for next event

Design rationale: Signal-based timers are more reliable than clock_gettime() comparison because they handle system clock adjustments and don’t drift under CPU load. Each sensor has an independent timer, allowing simultaneous detection on different holes.

### 4.1.4	Scoring Dispatch

The scoring chain processes a validated, debounced pin event:

// Simplified flow (actual code is inline in logic_handle_events)
Player* player = &players[current_player_index];

if (player->detection_count < SENSORS_PER_TURN) {
    switch (current_game_mode) {
        case GAME_MODE_STROKE_PLAY:
            stroke_play_process_pin(player, index, pin, &leds);
            break;
        case GAME_MODE_MATCH_PLAY:
            // Inline: player->round_total_score += score
            break;
        case GAME_MODE_QUOTA:
            quota_play_process_pin(player, index, pin, &leds);
            player->round_total_score = player->score;
            break;
        case GAME_MODE_VEGAS:
            vegas_quota_play_process_pin(player, index, pin, &leds,
                                         players, num_players);
            player->round_total_score = player->score;
            break;
    }
    player->detection_count++;
    logic_update_label_text(...);
    // ... completion checks, turn advancement
}

### 4.1.5	Turn Management

Turn advancement follows this pattern:

| Condition | Action |
| --- | --- |
| detection_count < SENSORS_PER_TURN | Continue current turn |
| detection_count == SENSORS_PER_TURN | Complete turn, advance player |
| Quota/Vegas: par3==0 && par4==0 && par5==0 | Immediate game end |
| Quota/Vegas 1P: after 1 detection | Reset count (no 2-ball requirement) |

Player rotation:

current_player_index = (current_player_index + 1) % num_players;

Turn cycling patterns for player highlights:

| Players | Detections per cycle | Pattern |
| --- | --- | --- |
| 2P | 8 (4 turns x 2 balls) | P1, P1, P2, P2, P1, P1, P2, P2 |
| 3P | 12 (6 turns x 2 balls) | P1, P1, P2, P2, P3, P3, repeat |
| 4P | 24 (12 turns x 2 balls) | P1, P1, P2, P2, P3, P3, P4, P4, repeat |

## 4.2	Game Mode Modules (modules/game_modes/)

### 4.2.1	Stroke Play (strokeplay.c)

Interface:

void stroke_play_process_pin(Player* player, int player_index,
                              unsigned int gpio_pin,
                              led_strip_controller_t* leds);

Algorithm:

- Null-check player (error) and leds (warn, continue)

- Switch on gpio_pin:

  - Pin 17: score += 3, green flash, “3 points” sound

  - Pin 26: score += 4, green flash, “4 points” sound

  - Pin 27: score += 5, green flash, “5 points” sound

  - Pin 24: score += 0, white flash, “0 points” sound

- Score is cumulative across all holes

Winner determination: After all holes completed, check_all_players_completed() compares players[i].score values. Highest score wins.

### 4.2.2	Match Play (matchplay.c + inline in gpio_event.c)

Scoring algorithm (per hole):

Both players play the hole (2 detections each)
    |
    +-> Compare round_total_score
    |   P1 > P2: players[0].upAndDown++, players[1].upAndDown--
    |   P2 > P1: players[0].upAndDown--, players[1].upAndDown++
    |   Equal:   no change (halved)
    |
    +-> Reset round_total_score for both
    +-> Advance both to next hole

Early victory condition:

int lead = max(players[0].upAndDown, players[1].upAndDown);
int holes_remaining = current_hole_mode - players[0].current_hole - 1;
bool early_victory = (lead > 0 && lead > holes_remaining);

If early_victory == true, the game ends immediately without playing remaining holes. The winner display uses “X&Y” format (e.g., “3&2” means 3 up with 2 holes remaining).

UI display format:

| State | Player 1 | Player 2 |
| --- | --- | --- |
| P1 leads by 2 | “2UP” | “2DN” |
| P2 leads by 1 | “1DN” | “1UP” |
| Even | “E” | “E” |

### 4.2.3	Quota Points (quotaplay.c)

Interface:

void quota_play_process_pin(Player* player, int player_index,
                             unsigned int gpio_pin,
                             led_strip_controller_t* leds);

Algorithm:

- Switch on gpio_pin:

  - Pin 17: score += 3, decrement par3_count if > 0

  - Pin 26: score += 4, decrement par4_count if > 0

  - Pin 27: score += 5, decrement par5_count if > 0

  - Pin 24: no score change, no par change

- LED flash and sound for all holes

Completion test:

int quota_player_completed(Player* player) {
    return (player->par3_count == 0 &&
            player->par4_count == 0 &&
            player->par5_count == 0);
}

### 4.2.4	Vegas Quota Points (quotaplay.c)

Interface:

void vegas_quota_play_process_pin(Player* player, int player_index,
                                   unsigned int gpio_pin,
                                   led_strip_controller_t* leds,
                                   Player* all_players,
                                   uint8_t num_players);

Bonus scoring algorithm:

For each category (3pt, 4pt, 5pt):
    Count how many players have closed it (par_count == 0)
    Category is "alive" if:
        - Single player mode (always alive), OR
        - Fewer than 2 players have closed it

On ball detection:
    If par_count > 0:
        Decrement par_count (countdown phase)
    Else if category alive:
        Add bonus points to score
    Else:
        No bonus (category dead)

Dead category rule (multiplayer):

int closed_par3 = 0;
for (uint8_t i = 0; i < num_players; i++)
    if (all_players[i].par3_count == 0) closed_par3++;

int par3_alive = (num_players == 1) || (closed_par3 < 2);

State diagram per category:

Player completes
    OPEN ---------> OPEN (1 closed)
                         |
                     2nd player completes
                         |
                         v
                    DEAD (no bonus for anyone)

## 4.3	Sound Module (modules/sound_logic/)

### 4.3.1	Interface

int         init_audio_system();          // SDL2_mixer init
void        cleanup_audio_system();       // Resource cleanup
Mix_Chunk*  load_sound_effect(const char* path);  // WAV loading
void        play_sound_effect(Mix_Chunk* sound);  // Immediate play
lv_timer_t* play_sound_once(Mix_Chunk* sound, uint32_t delay_ms); // Delayed
void        stop_all_sounds();
void        mute_audio();
void        unmute_audio();
int         is_audio_muted();

### 4.3.2	Non-Blocking Playback Design

Sound playback must not block the 5ms main loop. The design uses LVGL’s timer system:

play_sound_once(sound, delay_ms)
    |
    +-> Create lv_timer with delay_ms period
    +-> Set repeat count = 1 (one-shot)
    +-> Store Mix_Chunk* as timer user_data
    +-> Return immediately
    |
    ... delay_ms later (during lv_timer_handler) ...
    |
    +-> Timer callback fires
    +-> Mix_PlayChannel(-1, sound, 0)
    +-> Timer self-deletes

### 4.3.3	Sound File Organization

All WAV files reside in modules/game_sounds/. The base path is configurable:

#ifndef SOUND_DIR
#define SOUND_DIR "modules/game_sounds/"    // Development default
#endif
// Override for Yocto: cmake -DSOUND_DIR_PATH="/usr/share/goball/sounds/"

### 4.3.4	Playback Macros

Two patterns are used:

Immediate (navigation sounds):

#define PLAY_BACKBTN_SOUND \
    if (!is_audio_muted()) \
        play_sound_effect(load_sound_effect(SOUND_BACKBTN_WAV));

Randomized with delay (winner announcements):

#define PLAY_PLAYER1WINS_WAV                          \
    do {                                              \
        const char* sounds[] = {                      \
            SOUND_PLAYER1WINS1_WAV,                   \
            SOUND_PLAYER1WINS2_WAV,                   \
            SOUND_PLAYER1WINS3_WAV,                   \
            SOUND_PLAYER1WINS4_WAV,                   \
            SOUND_PLAYER1WINS5_WAV                    \
        };                                            \
        int idx = rand() % 5;                         \
        Mix_Chunk* s = load_sound_effect(sounds[idx]);\
        if (s) play_sound_once(s, SOUND_DELAY_PLAYER_WINS_MS); \
    } while (0)

### 4.3.5	Timing Configuration

| Constant | Default | Purpose |
| --- | --- | --- |
| SOUND_DELAY_PLAYER_ANNOUNCE_MS | 750 ms | “Player X” turn announcement |
| SOUND_DELAY_TEAM_ANNOUNCE_MS | 750 ms | “Team X” turn announcement |
| SOUND_DELAY_PLAYER_WINS_MS | 750 ms | “Player X wins!” announcement |
| SOUND_DELAY_TEAM_WINS_MS | 750 ms | “Team X wins!” announcement |
| SOUND_DELAY_TURN_SWITCH_MS | 500 ms | Next player/team announcement |

## 4.4	LED Module (modules/led_logic/)

### 4.4.1	Initialization with Graceful Degradation

init_led_controller(gpio1=3, gpio2=2)
    |
    +-> Set defaults (brightness=10)
    +-> pio_open(0)
    |   FAIL (PIO_IS_ERR) -> enabled=false, return
    |   OK -> continue
    +-> pio_claim_unused_sm() x2
    |   FAIL (sm < 0) -> enabled=false, return
    |   OK -> continue
    +-> pio_sm_config_xfer() x2
    +-> pio_add_program(&ws2812_program)
    +-> pio_sm_clear_fifos() x2
    +-> pio_sm_set_clkdiv(1.0) x2
    +-> ws2812_program_init() x2 (800kHz, non-RGBW)
    +-> initialize_train_positions()
    +-> enabled = true

Key design point: Uses pio_open() + PIO_IS_ERR() instead of the pio0 macro (which calls pio_open_helper() that exit(1) on failure). This allows the application to continue without LEDs on non-RPi5 hardware.

### 4.4.2	Train Animation Algorithm

The idle animation displays colored “trains” traveling along the LED strip:

void update_led_animation(led_strip_controller_t* controller) {
    if (!controller->enabled || animation_paused) return;

    // 1. Fill both strips with background (white, brightness-scaled)
    update_led_strip(controller, databuf1, train_positions, ...);
    update_led_strip(controller, databuf2, train_positions, ...);

    // 2. Push data to hardware
    pio_sm_xfer_data(pio, sm1, databuf1);
    pio_sm_xfer_data(pio, sm2, databuf2);

    // 3. Advance all train positions
    for (t = 0; t < NUM_TRAINS; t++)
        train_positions[t] = (train_positions[t] + 1) % PIXELS;
}

Train initialization: Trains are evenly spaced:

train_positions[i] = (i * PIXELS) / NUM_TRAINS;
// For PIXELS=144, NUM_TRAINS=5: positions = {0, 28, 57, 86, 115}

Per-pixel rendering in update_led_strip():

- Set all pixels to white (background) at current brightness

- For each train, overwrite TRAIN_LENGTH=10 consecutive pixels with train color (dark green: W=0x00, B=0x00, R=0x08, G=0x65)

### 4.4.3	Flash Effect Design

trigger_flash_with_color(controller, duration_ms, color)
    |
    +-> animation_paused = true
    +-> clear_all_leds()
    +-> malloc(flash_context_t) with color
    +-> Create flash_timer (50ms interval, duration/250 cycles)
    |       +-> flash_toggle(): alternates between color and off
    +-> Create restore_timer (duration_ms, one-shot)
            +-> restore_animation(): animation_paused = false

Flash timing example (1000ms green flash for scoring):

- Flash timer: 50ms interval, 4 cycles = 200ms of toggling

- Restore timer: 1000ms, resumes animation

- Visual: rapid on/off blinks, then animation resumes

### 4.4.4	Brightness Scaling

uint8_t scale_brightness(uint8_t color, uint8_t brightness) {
    return (color * brightness) / 255;
}

Linear scaling. At brightness=50 (default), a full-intensity channel (0xFF) outputs (255 * 50) / 255 = 50.

## 4.5	Debug Module (modules/debug/)

### 4.5.1	Log Level Hierarchy

typedef enum {
    DEBUG_LEVEL_NONE  = 0,  // No output
    DEBUG_LEVEL_ERROR = 1,  // Unrecoverable errors
    DEBUG_LEVEL_WARN  = 2,  // Handled anomalies
    DEBUG_LEVEL_INFO  = 3,  // Operational milestones
    DEBUG_LEVEL_DEBUG = 4,  // Diagnostic detail
    DEBUG_LEVEL_TRACE = 5   // Per-frame, function entry/exit
} debug_level_t;

### 4.5.2	Compile-Time Elimination

Each log macro is wrapped in a preprocessor guard:

#if DEBUG_LEVEL >= DEBUG_LEVEL_INFO
#define DEBUG_INFO(module, format, ...) \
    DEBUG_PRINT(DEBUG_LEVEL_INFO, DEBUG_MODULE_NAMES[module], \
                format, ##__VA_ARGS__)
#else
#define DEBUG_INFO(module, format, ...) ((void)0)
#endif

In Release builds (DEBUG_LEVEL=2), DEBUG_INFO, DEBUG_DEBUG, and DEBUG_TRACE compile to no-ops with zero runtime cost.

### 4.5.3	Module Tagging

typedef enum {
    MODULE_MAIN, MODULE_LVGL, MODULE_UI, MODULE_GAME,
    MODULE_SOUND, MODULE_LED, MODULE_GPIO, MODULE_INPUT,
    MODULE_ANIMATION, MODULE_LOGIC, MODULE_HAL,
    MODULE_COUNT
} debug_module_t;

Each log line includes the module name for filtering:

[INFO ] [GAME ] Player 1 scored 3 points
[DEBUG] [LED  ] Flash timer created: interval=50ms, cycles=4
[WARN ] [GPIO ] PIO hardware not available - LEDs disabled

### 4.5.4	Features

- ANSI colors: Error=red, Warn=yellow, Info=green, Debug=cyan, Trace=magenta (toggle: ENABLE_DEBUG_COLORS)

- Timestamps: Millisecond precision via clock_gettime(CLOCK_MONOTONIC) (toggle: ENABLE_DEBUG_TIMESTAMP)

- File and line: Each log includes __FILE__:__LINE__ for source tracing

## 4.6	UI Module (modules/ui_logic/ + ui/)

### 4.6.1	Two-Layer Event Architecture

Layer 1: SquareLine Studio generated (ui/ui.c)

- Screen definitions and widget tree construction

- Button event callbacks for navigation (_ui_screen_change())

- Sound playback macros on button press

- Approximately 7400 lines, auto-generated

Layer 2: Custom logic (modules/ui_logic/ui_logic.event.c)

- Game mode setup from UI selections (player count, hole count)

- Live score display updates

- Player highlighting (background color changes)

- Crown icon visibility for winners

- Scorecard population

- Screen transitions on game completion

- Approximately 6500 lines

### 4.6.2	Widget Naming Convention

SquareLine Studio generates widget names with encoded metadata:

ui_[Mode][Players][Holes][Screen][Widget][Detail]

Examples:
  ui_SP2P9HGSBCPText     Stroke Play, 2P, 9H, Game Screen, Ball Counter
  ui_MP1V19HScP1SText3   Match Play, 1v1, 9H, Scorecard, P1, Hole 3
  ui_VQ3P18HGSBSText     Vegas Quota, 3P, 18H, Game Screen, Bonus Score
  ui_Q4P9HGSMMButton     Quota, 4P, 9H, Game Screen, Main Menu Button

### 4.6.3	Score Reset Design

reset_scores() resets all UI widgets to initial state:

- Stroke Play scorecards: all cells to “0”

- Match Play scorecards: all cells to “-”

- Ball counters: reset to “2”

- Crown icons: hidden (lv_obj_add_flag(LV_OBJ_FLAG_HIDDEN))

- Bonus score labels (Vegas): reset to “0”

- Player highlights: reset to default colors

### 4.6.4	Player Highlighting

Background colors cycle based on detection count:

| Color | Hex | Usage |
| --- | --- | --- |
| Active (bright) | 0x34bb19 | Current player’s turn |
| Active (dim) | 0x5AB66A | Secondary highlight |
| Inactive | Default | Not current player’s turn |

# 5	Interface Design

## 5.1	Hardware Interfaces

### 5.1.1	GPIO Sensor Interface

// Pin mapping
unsigned int sensor_pins[NUM_SENSORS] = {17, 26, 27, 24};

// Initialization
struct gpiod_chip*     chip;   // /dev/gpiochip0
struct gpiod_line_bulk lines;  // Bulk line handle

// Event detection
gpiod_line_event_wait_bulk(&lines, &(struct timespec){0, 0}, &event_lines);
// Non-blocking: timeout = {0, 0}
// Returns: >0 if events pending, 0 if none, <0 on error

### 5.1.2	PIO LED Interface

// Initialization
PIO pio = pio_open(0);                           // Open PIO device
int sm  = pio_claim_unused_sm(pio, false);       // Claim state machine
int off = pio_add_program(pio, &ws2812_program); // Load WS2812 program

// Data transfer (per frame)
pio_sm_xfer_data(pio, sm, PIO_DIR_TO_SM, sizeof(databuf), databuf);
// Sends PIXELS*4 bytes to state machine for serial output

## 5.2	Software Interfaces

### 5.2.1	Module Interface Summary

| Function | Module | Called By | Purpose |
| --- | --- | --- | --- |
| logic_initialize_game() | gpio_event | main | Setup game state + GPIO |
| logic_handle_events() | gpio_event | main | Process sensor events |
| init_audio_system() | sound_logic | main | SDL2_mixer init |
| init_led_controller() | led_logic | main | PIO + LED init |
| update_led_animation() | led_logic | main | Per-frame LED update |
| set_game_mode() | gpio_event | ui_logic | UI game mode selection |
| set_hole_mode() | gpio_event | ui_logic | UI hole count selection |
| set_num_players() | gpio_event | ui_logic | UI player count selection |
| reset_scores() | gpio_event | ui_logic | Reset all game state + UI |
| stroke_play_process_pin() | strokeplay | gpio_event | Score a stroke play event |
| quota_play_process_pin() | quotaplay | gpio_event | Score a quota event |
| vegas_quota_play_process_pin() | quotaplay | gpio_event | Score a vegas event |
| trigger_flash_with_color() | led_logic | game modes | LED flash effect |
| play_sound_once() | sound_logic | gpio_event | Delayed sound playback |
| check_all_players_completed() | gpio_event | gpio_event | End-of-game detection |

# 6	Error Handling Design

## 6.1	Error Handling Strategy

The system uses a fail-soft approach: hardware failures disable the affected subsystem while allowing the rest of the application to continue.

| Subsystem | Failure Mode | Handling | User Impact |
| --- | --- | --- | --- |
| GPIO | /dev/gpiochip0 not found | gpio_available = 0 | UI-only mode (no scoring) |
| PIO | PIO device unavailable | leds.enabled = false | No LED feedback |
| PIO | State machines unavailable | leds.enabled = false | No LED feedback |
| Audio | SDL2_mixer init fails | Warning logged | No sound (game still works) |
| Sound file | WAV not found | load_sound_effect() returns NULL | Silent (no crash) |
| Debounce timer | Timer creation fails | logic_initialize_game() returns -1 | Falls back to UI-only |

## 6.2	Guard Patterns

LED guard (every LED function):

void set_all_leds(led_strip_controller_t* controller, uint32_t color) {
    if (!controller->enabled) return;  // Early exit if no hardware
    // ... proceed with PIO operations
}

Sensor guard (main loop):

if (gpio_available) {
    logic_handle_events(&event_lines, &event, num_players);
}

Sound guard (all playback macros):

if (!is_audio_muted())
    play_sound_effect(load_sound_effect(SOUND_PATH));
// load_sound_effect returns NULL on missing file
// play_sound_effect handles NULL gracefully

Null parameter guards (game mode functions):

if (player == NULL) {
    DEBUG_ERROR(MODULE_GAME, "Invalid player pointer (NULL)");
    return;
}
if (leds == NULL) {
    DEBUG_WARN(MODULE_GAME, "LED controller is NULL - visual feedback disabled");
    // Continue without LED effects
}

# 7	Performance Design

## 7.1	Timing Budget

The main loop targets a 5ms cycle (200 Hz):

| Step | Operation | Typical Time | Blocking? |
| --- | --- | --- | --- |
| 1 | update_led_animation() | < 0.5ms | No (PIO DMA) |
| 2 | logic_handle_events() | < 0.1ms | No (non-blocking poll) |
| 3 | lv_timer_handler() | 1-4ms | Partial (rendering) |
| 4 | usleep(5000) | 5ms | Yes (intentional) |
| Total |  | ~6-10ms |  |

## 7.2	Memory Budget

| Component | Allocation | Size |
| --- | --- | --- |
| LVGL heap | LV_MEM_SIZE | 512 MB |
| LVGL image cache | LV_CACHE_DEF_SIZE | 8 MB |
| LVGL draw buffer | LV_DRAW_LAYER_SIMPLE_BUF_SIZE | 16 MB |
| LED data buffers | PIXELS * 4 * 2 | 1,152 bytes |
| Player state | sizeof(Player) * 4 | ~200 bytes |
| Flash context | malloc per flash | ~12 bytes (freed after) |
| Sound chunks | SDL2_mixer managed | Variable (WAV PCM data) |

## 7.3	Rendering Performance

| Setting | Value | Impact |
| --- | --- | --- |
| Draw units | 4 parallel (LV_DRAW_SW_DRAW_UNIT_CNT) | Multi-core rendering |
| Render mode | Direct (LV_SDL_RENDER_MODE_DIRECT) | Zero-copy to SDL |
| Buffer count | 1 (LV_SDL_BUF_COUNT) | Single buffer, no tearing issues at 62.5 Hz |
| DPI | 130 (LV_DPI_DEF) | Touch accuracy calibration |

# 8	Build Configuration Design

## 8.1	Cross-Compilation Toolchain

toolchain-aarch64.cmake
    |
    +-> CMAKE_SYSTEM_NAME = Linux
    +-> CMAKE_SYSTEM_PROCESSOR = aarch64
    +-> CMAKE_C_COMPILER = aarch64-linux-gnu-gcc
    +-> CMAKE_SYSROOT = ${CMAKE_CURRENT_LIST_DIR}/rpi5-sysroot
    +-> CMAKE_FIND_ROOT_PATH_MODE_LIBRARY = ONLY
    +-> CMAKE_FIND_ROOT_PATH_MODE_INCLUDE = ONLY
    +-> PKG_CONFIG_SYSROOT_DIR = ${CMAKE_SYSROOT}

Sysroot resolution: Defaults to rpi5-sysroot/ relative to the toolchain file. Override with -DSYSROOT_PATH=/custom/path.

## 8.2	Debug Level Configuration

CMakeLists.txt
    |
    +-> CMAKE_BUILD_TYPE = Debug
    |       +-> DEBUG=1, DEBUG_LEVEL=4 (DEBUG)
    |       +-> ENABLE_DEBUG_TRACE=ON -> DEBUG_LEVEL=5 (TRACE)
    |
    +-> CMAKE_BUILD_TYPE = Release
            +-> DEBUG_LEVEL=2 (WARN only)

## 8.3	Source File Discovery

CMake uses GLOB_RECURSE to discover source files per module:

FILE(GLOB_RECURSE LVGL_Sources       CONFIGURE_DEPENDS lvgl/*.c)
FILE(GLOB_RECURSE UI_Sources         CONFIGURE_DEPENDS ui/*.c ui/*.cpp)
FILE(GLOB_RECURSE LOGIC_Sources      CONFIGURE_DEPENDS modules/logic/*.c)
FILE(GLOB_RECURSE UI_LOGIC_Sources   CONFIGURE_DEPENDS modules/ui_logic/*.c)
FILE(GLOB_RECURSE SOUND_LOGIC_Sources CONFIGURE_DEPENDS modules/sound_logic/*.c)
FILE(GLOB_RECURSE LED_LOGIC_Sources  CONFIGURE_DEPENDS modules/led_logic/*.c)
FILE(GLOB_RECURSE GAME_MODE_Sources  CONFIGURE_DEPENDS modules/game_modes/*.c)
FILE(GLOB_RECURSE DEBUG_Sources      CONFIGURE_DEPENDS modules/debug/*.c)

CONFIGURE_DEPENDS ensures CMake re-runs when new source files are added.

## 8.4	Link Dependencies

SquareLine_Project (executable)
    |
    +-> SDL2            (display, input)
    +-> SDL2_mixer      (audio)
    +-> pio             (LED PIO control, from utils/piolib/)
    +-> libgpiod        (GPIO sensor access)
    +-> m               (math library)

# 9	Appendix A: Sensor Pin Mapping

| Pin | Score | Sound File | LED Color | Debounce |
| --- | --- | --- | --- | --- |
| GPIO 17 | 3 points | threePoints.wav | Green | 3000 ms |
| GPIO 26 | 4 points | fourPoints.wav | Green | 3000 ms |
| GPIO 27 | 5 points | fivePoints.wav | Green | 3000 ms |
| GPIO 24 | 0 points | zeroPoints.wav | White | 3000 ms |

# 10	Appendix B: Sound File Inventory

## 10.1	Navigation Sounds

| File | Trigger |
| --- | --- |
| backBtn.wav | Back button press |
| mainMenu.wav | Main menu button press |
| home.wav | Home navigation |
| scoreCard.wav | Scorecard view |

## 10.2	Mode Selection Sounds

| File | Trigger |
| --- | --- |
| strokePlay.wav | Stroke Play mode selected |
| matchPlay.wav | Match Play mode selected |
| quotaPoints.wav | Quota Points mode selected |
| vegasQuotaPoints.wav | Vegas Quota selected |
| onePlayer.wav - fourPlayers.wav | Player count selected |
| nineHoles.wav, eighteenHoles.wav | Hole count selected |
| oneVOne.wav, twoVTwo.wav | Match Play mode selected |

## 10.3	Scoring Sounds

| File | Trigger |
| --- | --- |
| threePoints.wav | 3-point hole detected |
| fourPoints.wav | 4-point hole detected |
| fivePoints.wav | 5-point hole detected |
| zeroPoints.wav | 0-point hole detected |

## 10.4	Turn Sounds

| File | Trigger |
| --- | --- |
| playerOne.wav - playerFour.wav | Player turn announcement |
| teamOne.wav, teamTwo.wav | Team turn announcement |

## 10.5	Winner Sounds (5 variants each)

| Pattern | Trigger |
| --- | --- |
| player1wins1.wav - player1wins5.wav | Player 1 wins (random pick) |
| player2wins1.wav - player2wins5.wav | Player 2 wins (random pick) |
| player3wins1.wav - player3wins5.wav | Player 3 wins (random pick) |
| player4wins1.wav - player4wins5.wav | Player 4 wins (random pick) |
| team1wins1.wav - team1wins5.wav | Team 1 wins (random pick) |
| team2wins1.wav - team2wins5.wav | Team 2 wins (random pick) |

# 11	Appendix C: LVGL Configuration Summary

| Parameter | Value | Notes |
| --- | --- | --- |
| LVGL Version | 9.1.0 |  |
| Color Depth | 32-bit (XRGB8888) |  |
| Memory Pool | 512 MB |  |
| Image Cache | 8 MB |  |
| Draw Layer Buffer | 16 MB |  |
| Refresh Period | 16 ms (62.5 Hz) |  |
| DPI | 130 |  |
| Draw Units | 4 (parallel SW) | Multi-core |
| Render Mode | Direct | Zero-copy to SDL |
| OS Support | None (bare loop) |  |
| Default Font | Montserrat 14 |  |
| Text Encoding | UTF-8 |  |
| Performance Monitor | Enabled | FPS/CPU overlay |
| Memory Monitor | Enabled | Memory usage overlay |
| Image Decoders | PNG, BMP, JPEG, GIF |  |