# GoBall - Build Environment Document

Yocto/Poky Custom Linux Distribution for Raspberry Pi 5

Ahmed Abdelghani

February 2026

Table of Contents

# 1	Introduction to the Yocto Project

## 1.1	What is Yocto?

The Yocto Project is an open-source collaboration framework that helps developers create custom Linux distributions for embedded systems. Instead of using a general-purpose distribution like Raspbian (which includes thousands of packages you don’t need), Yocto builds a minimal, purpose-built Linux image containing only what your application requires.

For GoBall, this means a Linux image that boots directly into the scoring application — no desktop environment, no unnecessary services, no login screen. Just power on and play.

## 1.2	Key Terminology

| Term | What It Is | Analogy |
| --- | --- | --- |
| Poky | The reference distribution of the Yocto Project. It includes BitBake, OpenEmbedded-Core metadata, and a default configuration. | The “starter kit” — everything you need to begin building |
| BitBake | The build engine (task executor). It reads recipes and executes build tasks (fetch, compile, install, package). | Like make, but for entire Linux distributions |
| OpenEmbedded (OE) | The build framework that provides the core recipes and classes. Poky bundles OE-Core. | The recipe library and build rules |
| Recipe (.bb) | A file that tells BitBake how to build one piece of software. Contains source URL, dependencies, compile instructions, and install rules. | A Makefile + package spec for one component |
| **Layer (meta-*)** | A directory containing related recipes, configuration, and classes. Layers are stacked — higher priority layers override lower ones. | Like plugin modules that add or customize functionality |
| Machine | A configuration defining the target hardware (CPU architecture, bootloader, kernel). For GoBall: raspberrypi5. | The “what hardware am I building for?” setting |
| Distro | A configuration defining the software policy (init system, package format, features). For GoBall: goball-distro. | The “what kind of Linux do I want?” setting |
| Image | The final output — a bootable filesystem image combining the kernel, root filesystem, and all installed packages. | The .img file you flash to an SD card |
| BSP | Board Support Package — a layer with hardware-specific kernel config, bootloader, and firmware. meta-raspberrypi is the RPi5 BSP. | Hardware drivers and boot configuration |
| bbappend | A file that modifies an existing recipe from another layer without editing the original. | A patch/override for someone else’s recipe |
| Sysroot | A staging area containing headers and libraries for cross-compilation. BitBake populates this automatically. | The target’s /usr/include and /usr/lib, but on the build host |

## 1.3	Why Yocto for GoBall?

| Approach | Boot Time | Image Size | Maintenance | Kiosk Suitability |
| --- | --- | --- | --- | --- |
| Raspbian (full desktop) | ~30s | 4+ GB | Package updates can break things | Poor — must strip down |
| Raspbian Lite + manual config | ~15s | 1+ GB | Manual dependency management | Medium |
| Yocto (goball-image) | ~8s | ~400 MB | Reproducible, version-controlled | Excellent — built for it |

## 1.4	The Build Workflow

[Source Code] --> [BitBake reads recipes] --> [Fetches sources]
     --> [Cross-compiles] --> [Packages] --> [Assembles rootfs]
     --> [Creates bootable image (.wic)]

Detailed steps:

- Fetch (do_fetch): Download source code from Git, tarballs, or local files

- Unpack (do_unpack): Extract archives into a work directory

- Patch (do_patch): Apply any patches

- Configure (do_configure): Run cmake/autotools/meson configure step

- Compile (do_compile): Cross-compile for aarch64

- Install (do_install): Install into a staging directory (fake root)

- Package (do_package): Create RPM/DEB/IPK packages

- Image (do_image): Combine all packages into a root filesystem image

# 2	Host System Requirements

## 2.1	Operating System

| Requirement | Recommended | Minimum |
| --- | --- | --- |
| OS | Ubuntu 22.04 LTS or 24.04 LTS | Any Linux with glibc 2.31+ |
| Architecture | x86_64 | x86_64 only |

## 2.2	Required Packages

Install on Ubuntu:

sudo apt install gawk wget git diffstat unzip texinfo gcc build-essential \
    chrpath socat cpio python3 python3-pip python3-pexpect xz-utils \
    debianutils iputils-ping python3-git python3-jinja2 python3-subunit \
    zstd liblz4-tool file locales libacl1 libsdl1.2-dev xterm

## 2.3	Hardware Requirements

| Resource | Minimum | Recommended | GoBall Build |
| --- | --- | --- | --- |
| CPU cores | 4 | 8+ | 28 threads configured |
| RAM | 8 GB | 16+ GB | 32 GB used |
| Disk space | 50 GB | 100+ GB | ~80 GB (downloads + sstate + build) |
| Disk type | HDD | SSD strongly recommended | NVMe SSD |

Build time estimates:

| Build Type | Time (8 cores) | Time (28 cores) |
| --- | --- | --- |
| First full build | ~4-6 hours | ~1-2 hours |
| Rebuild after app change | ~5-10 minutes | ~2-3 minutes |
| Rebuild with sstate cache | ~10-20 minutes | ~5 minutes |

## 2.4	Target Hardware

- Raspberry Pi 5 (BCM2712, Cortex-A76, 4/8 GB RAM)

- microSD card (32 GB minimum, Class 10 / A2 recommended)

- 7” touchscreen display (2560x720 via HDMI)

- 4x IR break-beam sensors

- 2x WS2812 LED strips (144 LEDs each)

- Speaker with amplifier

# 3	Poky Directory Structure

The GoBall build environment lives at /home/q/yocto/goball/poky/:

poky/
+-- bitbake/              BitBake build engine
+-- meta/                 OpenEmbedded-Core recipes (base system)
+-- meta-poky/            Poky reference distro configuration
+-- meta-yocto-bsp/       Yocto reference BSP (generic hardware)
+-- meta-raspberrypi/     Raspberry Pi BSP (kernel, firmware, boot)
+-- meta-openembedded/    Community recipes (networking, multimedia, etc.)
|   +-- meta-oe/          General-purpose recipes
|   +-- meta-python/      Python packages
|   +-- meta-multimedia/  Audio/video libraries (PulseAudio, GStreamer)
|   +-- meta-networking/  NetworkManager, wpa_supplicant, etc.
+-- meta-goball/          *** Custom GoBall layer ***
+-- build/                Build output directory
|   +-- conf/             Build configuration (local.conf, bblayers.conf)
|   +-- tmp-glibc/        Build artifacts, sysroot, deployed images
+-- oe-init-build-env     Script to initialize the build environment
+-- scripts/              BitBake helper scripts

### 3.0.1	Key Directories Explained

bitbake/ — The BitBake build engine itself. You never edit files here. It parses recipes, resolves dependencies, and orchestrates the build.

meta/ — OpenEmbedded-Core. Contains foundational recipes for the C library (glibc), coreutils, systemd, GCC, busybox, and hundreds of base packages. This is the “operating system” layer.

meta-raspberrypi/ — The RPi BSP layer. Provides the raspberrypi5 machine configuration, the linux-raspberrypi kernel recipe, GPU firmware, VideoCore drivers, and RPi-specific device tree overlays.

meta-openembedded/ — Community-maintained recipes. GoBall uses:

- meta-oe: General tools (i2c-tools, devmem2, htop)

- meta-multimedia: PulseAudio, ALSA plugins

- meta-networking: NetworkManager, wpa_supplicant

- meta-python: Python support (for build tools)

meta-goball/ — The custom layer. Contains everything specific to the GoBall project. This is the only layer you edit. Covered in detail in Section 5.

oe-init-build-env — A shell script you source to set up the build environment. It creates the build/ directory and sets environment variables for BitBake.

# 4	Build Configuration

## 4.1	bblayers.conf — Layer Stack

File: build/conf/bblayers.conf

POKY_BBLAYERS_CONF_VERSION = "2"

BBPATH = "${TOPDIR}"
BBFILES ?= ""

BBLAYERS ?= " \
  /home/q/yocto/goball/poky/meta \
  /home/q/yocto/goball/poky/meta-poky \
  /home/q/yocto/goball/poky/meta-yocto-bsp \
  /home/q/yocto/goball/poky/meta-raspberrypi \
  /home/q/yocto/goball/poky/meta-openembedded/meta-oe \
  /home/q/yocto/goball/poky/meta-openembedded/meta-python \
  /home/q/yocto/goball/poky/meta-openembedded/meta-multimedia \
  /home/q/yocto/goball/poky/meta-openembedded/meta-networking \
  /home/q/yocto/goball/poky/meta-goball \
  "

Line-by-line:

| Variable | Purpose |
| --- | --- |
| POKY_BBLAYERS_CONF_VERSION | Config file format version. Must be “2” for current Poky. |
| BBPATH | BitBake search path. ${TOPDIR} is the build/ directory. |
| BBFILES | List of recipe files. Each layer appends to this via its layer.conf. |
| BBLAYERS | Ordered list of active layers. Order matters — later layers can override earlier ones. |

Why each layer is included:

| Layer | Reason |
| --- | --- |
| meta | Core OS: glibc, coreutils, systemd, gcc, cmake |
| meta-poky | Poky reference distro defaults |
| meta-yocto-bsp | Generic BSP reference (required by Poky) |
| meta-raspberrypi | RPi5 machine config, kernel, firmware, VC4 GPU |
| meta-oe | htop, nano, i2c-tools, devmem2 |
| meta-python | Python dependencies for build tools |
| meta-multimedia | PulseAudio, ALSA plugins |
| meta-networking | NetworkManager, wpa_supplicant, iw |
| meta-goball | GoBall app, custom distro, device config, SDL2_mixer recipe |

## 4.2	local.conf — Build Settings

File: build/conf/local.conf

MACHINE = "raspberrypi5"
DISTRO = "goball-distro"
INIT_MANAGER = "systemd"

# GPU / display
ENABLE_DWC2_PERIPHERAL = "0"
DISABLE_VC4GRAPHICS = "0"
VC4DTBO = "vc4-kms-v3d"

# HDMI configuration
HDMI_FORCE_HOTPLUG = "1"
RPI_EXTRA_CONFIG = " \nkernel=Image \nhdmi_group=2 \nhdmi_mode=87 \
    \nhdmi_cvt=2560 720 60 3 0 0 0 \ndisable_splash=1 \n"

# Clean boot: no logos, no boot messages, no cursor, no splash
DISABLE_SPLASH = "1"
DISABLE_RPI_BOOT_LOGO = "1"
BOOT_DELAY = "0"
BOOT_DELAY_MS = "0"
CMDLINE:append = " console=tty3 quiet loglevel=0 \
    vt.global_cursor_default=0 video=HDMI-A-2:2560x720@60D"

# Disable U-Boot (causes black screen on RPi5)
RPI_USE_U_BOOT = "0"

# PIO kernel module for WS2812 LEDs
MACHINE_EXTRA_RRECOMMENDS += "kernel-module-rp1-pio"

# Extra disk space (1GB)
IMAGE_ROOTFS_EXTRA_SPACE = "1048576"

# Accept all licenses
LICENSE_FLAGS_ACCEPTED = "commercial synaptics-killswitch"

PACKAGE_CLASSES = "package_rpm"

# Development: allow root login with password
EXTRA_IMAGE_FEATURES += "debug-tweaks"

### 4.2.1	Variable-by-Variable Explanation

| Variable | Value | What It Does |
| --- | --- | --- |
| MACHINE | "raspberrypi5" | Target hardware. Tells BitBake to use the RPi5 kernel, firmware, and device tree from meta-raspberrypi. |
| DISTRO | "goball-distro" | Use our custom distro policy (defined in meta-goball/conf/distro/goball-distro.conf). Sets systemd, PulseAudio, Wayland. |
| INIT_MANAGER | "systemd" | Use systemd as PID 1 (not SysVinit). Required for service management (goball.service, weston.service). |
| ENABLE_DWC2_PERIPHERAL | "0" | Disable USB OTG peripheral mode (not needed, prevents kernel warnings). |
| DISABLE_VC4GRAPHICS | "0" | Keep VC4 GPU enabled (= 0 means “don’t disable”). Required for OpenGL/Wayland. |
| VC4DTBO | "vc4-kms-v3d" | Load the KMS/DRM device tree overlay for the GPU. Required for Weston/SDL2 display. |
| HDMI_FORCE_HOTPLUG | "1" | Force HDMI output even if no monitor is detected at boot. Prevents blank screen on late monitor power-on. |
| RPI_EXTRA_CONFIG | (see below) | Appended to config.txt on the boot partition. Configures HDMI resolution. |
| DISABLE_SPLASH | "1" | Hide the Raspberry Pi rainbow splash screen on boot. |
| DISABLE_RPI_BOOT_LOGO | "1" | Hide the Raspberry Pi logo from the kernel boot screen. |
| BOOT_DELAY / BOOT_DELAY_MS | "0" | No boot delay — start immediately. |
| CMDLINE:append | (see below) | Appended to the kernel command line. |
| RPI_USE_U_BOOT | "0" | Boot the kernel directly (no U-Boot). U-Boot causes a black screen on RPi5 with custom HDMI modes. |
| MACHINE_EXTRA_RRECOMMENDS | "kernel-module-rp1-pio" | Include the RP1 PIO kernel module. Required for WS2812 LED strip control. |
| IMAGE_ROOTFS_EXTRA_SPACE | "1048576" | Add 1 GB of free space to the root filesystem (for logs, updates, etc.). |
| LICENSE_FLAGS_ACCEPTED | "commercial synaptics-killswitch" | Accept commercial licenses for firmware blobs (WiFi, GPU). |
| PACKAGE_CLASSES | "package_rpm" | Use RPM format for packages (also supports deb, ipk). |
| EXTRA_IMAGE_FEATURES | "debug-tweaks" | Development convenience: allows root login without password, enables debug tools. Remove for production. |

### 4.2.2	HDMI Configuration Explained

The RPI_EXTRA_CONFIG variable adds these lines to config.txt:

kernel=Image              # Boot the uncompressed kernel image
hdmi_group=2              # CEA/DMT group 2 = DMT (monitor modes)
hdmi_mode=87              # Custom mode (defined by hdmi_cvt)
hdmi_cvt=2560 720 60 3 0 0 0  # 2560x720 @ 60Hz, aspect ratio 16:9
disable_splash=1          # No rainbow screen

The CMDLINE:append adds:

console=tty3              # Redirect kernel messages to tty3 (not visible)
quiet                     # Suppress most kernel boot messages
loglevel=0                # Only show KERN_EMERG messages
vt.global_cursor_default=0  # Hide the blinking text cursor
video=HDMI-A-2:2560x720@60D  # Force HDMI-A-2 at 2560x720, 60Hz

# 5	The meta-goball Custom Layer

## 5.1	Directory Structure

meta-goball/
+-- conf/
|   +-- layer.conf                    Layer metadata
|   +-- distro/
|       +-- goball-distro.conf        Custom distro definition
+-- build-templates/
|   +-- local.conf.sample             Template for build/conf/local.conf
|   +-- bblayers.conf.sample          Template for build/conf/bblayers.conf
+-- recipes-core/
|   +-- images/
|   |   +-- goball-image.bb           Root filesystem image recipe
|   +-- psplash/
|   |   +-- psplash_%.bbappend        Custom boot splash screen
|   |   +-- files/
|   |       +-- psplash-goball.png    GoBall splash image (2560x720)
|   |       +-- framebuf.conf         RPi5 framebuffer fix
|   +-- dropbear/
|       +-- dropbear_%.bbappend       SSH server configuration
|       +-- dropbear/
|           +-- dropbear              PAM config
|           +-- dropbear.default      Runtime options
+-- recipes-goball/
|   +-- goball/
|       +-- goball_1.0.bb             *** Main application recipe ***
|       +-- files/
|           +-- goball.service        systemd service unit
|           +-- pulseaudio-system.service  PulseAudio system service
+-- recipes-config/
|   +-- goball-config/
|       +-- goball-config_1.0.bb      Device configuration recipe
|       +-- files/
|           +-- 99-pio.rules          PIO udev rules
|           +-- authorized_keys       SSH public keys
|           +-- Banhof.nmconnection   WiFi profile
|           +-- Ethernet.nmconnection Ethernet profile (static IP)
|           +-- rpi-eeprom-setup.sh   EEPROM configuration script
|           +-- rpi-eeprom-setup.service  First-boot EEPROM service
+-- recipes-graphics/
|   +-- wayland/
|       +-- weston-init.bbappend      Weston kiosk mode config
|       +-- weston-init/
|           +-- weston.ini            Weston compositor config
|           +-- weston.service        Weston systemd service
+-- recipes-multimedia/
|   +-- pulseaudio/
|   |   +-- pulseaudio_%.bbappend     PulseAudio access fix
|   +-- sdl2-mixer/
|       +-- libsdl2-mixer_2.8.1.bb    SDL2_mixer recipe
+-- recipes-support/
|   +-- libgpiod/
|       +-- libgpiod_1.6.5.bb         libgpiod recipe
|       +-- files/
|           +-- libgpiod-1.6.5/       Full libgpiod source tree
+-- deploy-config.conf                Deployment parameter reference
+-- setup-goball.sh                   One-command build setup script
+-- README.md                         Layer documentation

## 5.2	conf/layer.conf — Layer Registration

# We have a conf and classes directory, add to BBPATH
BBPATH .= ":${LAYERDIR}"

# We have recipes-* directories, add to BBFILES
BBFILES += "${LAYERDIR}/recipes-*/*/*.bb \
            ${LAYERDIR}/recipes-*/*/*.bbappend"

BBFILE_COLLECTIONS += "meta-goball"
BBFILE_PATTERN_meta-goball = "^${LAYERDIR}/"
BBFILE_PRIORITY_meta-goball = "10"

LAYERDEPENDS_meta-goball = "core raspberrypi"
LAYERSERIES_COMPAT_meta-goball = "scarthgap"

Line-by-line:

| Line | Purpose |
| --- | --- |
| BBPATH .= ":${LAYERDIR}" | Adds this layer to BitBake’s search path for configuration files and classes. |
| BBFILES += "..." | Tells BitBake where to find recipes (.bb) and appends (.bbappend) in this layer. The glob recipes-*/*/*.bb matches all recipes in the standard directory structure. |
| BBFILE_COLLECTIONS += "meta-goball" | Registers this layer’s name for conflict detection and priority resolution. |
| BBFILE_PATTERN_meta-goball | Regex matching files from this layer. Used to assign priority. |
| BBFILE_PRIORITY_meta-goball = "10" | Priority 10 (higher than default 5). Our recipes override OE-Core defaults when there’s a conflict. |
| LAYERDEPENDS_meta-goball = "core raspberrypi" | This layer requires meta (core) and meta-raspberrypi to be present. BitBake errors if they’re missing. |
| LAYERSERIES_COMPAT_meta-goball = "scarthgap" | Compatible with the Scarthgap release of Yocto (April 2024). Prevents accidental use with incompatible Yocto versions. |

## 5.3	conf/distro/goball-distro.conf — Custom Distribution

DISTRO = "goball-distro"
DISTRO_NAME = "GoBall OS"
DISTRO_VERSION = "1.0"

DISTRO_FEATURES = "alsa pulseaudio systemd usbhost opengl wayland pam"
DISTRO_FEATURES:remove = "x11"
DISTRO_FEATURES_BACKFILL_CONSIDERED:append = " sysvinit"

VIRTUAL-RUNTIME_init_manager = "systemd"
VIRTUAL-RUNTIME_initscripts = "systemd-compat-units"

Line-by-line:

| Variable | Purpose |
| --- | --- |
| DISTRO / DISTRO_NAME / DISTRO_VERSION | Identity strings. Appear in /etc/os-release on the target. |
| DISTRO_FEATURES | The set of system capabilities enabled across all packages: |
| – alsa | Low-level audio drivers (ALSA kernel interface) |
| – pulseaudio | PulseAudio sound server (provides mixing, per-app volume) |
| – systemd | systemd init system and service management |
| – usbhost | USB host support (for keyboards, USB audio, etc.) |
| – opengl | OpenGL ES support (required by Weston compositor) |
| – wayland | Wayland display protocol (replaces X11) |
| – pam | Pluggable Authentication Modules (for SSH login) |
| DISTRO_FEATURES:remove = "x11" | Explicitly remove X11. GoBall uses Wayland/Weston only. |
| DISTRO_FEATURES_BACKFILL_CONSIDERED | Tells BitBake not to automatically add sysvinit (we use systemd). |
| VIRTUAL-RUNTIME_init_manager | Set systemd as the init system (PID 1). |
| VIRTUAL-RUNTIME_initscripts | Use systemd compatibility scripts instead of SysVinit scripts. |

## 5.4	recipes-core/images/goball-image.bb — The Image Recipe

This is the top-level recipe that defines what goes into the final SD card image.

SUMMARY = "GoBall Kiosk Image for Raspberry Pi 5"

IMAGE_FEATURES += "ssh-server-openssh splash"

inherit core-image extrausers

# Set root password to "123"
EXTRA_USERS_PARAMS = "usermod -p '\$5\$goball\$3Wgxx...T3U5' root;"

# Disable getty on tty1 - Weston uses it for kiosk display
disable_getty() {
    ln -sf /dev/null \
        ${IMAGE_ROOTFS}${systemd_system_unitdir}/getty@tty1.service
}
ROOTFS_POSTPROCESS_COMMAND += "disable_getty;"

IMAGE_INSTALL += " \
    goball \
    libsdl2 \
    libsdl2-mixer \
    libgpiod \
    libgpiod-tools \
    goball-config \
    weston \
    weston-init \
    mesa-megadriver \
    libegl-mesa \
    libgles2-mesa \
    libgbm \
    pulseaudio \
    pulseaudio-server \
    pulseaudio-module-alsa-sink \
    pulseaudio-module-alsa-source \
    alsa-utils \
    alsa-plugins \
    kernel-modules \
    rpi-eeprom \
    rpi-gpio \
    i2c-tools \
    devmem2 \
    nano \
    htop \
    networkmanager \
    networkmanager-nmcli \
    wpa-supplicant \
    linux-firmware-rpidistro-bcm43455 \
    iw \
"

### 5.4.1	Package Groups Explained

Core Application:

| Package | Why |
| --- | --- |
| goball | The GoBall application binary + sound files |
| libsdl2 | Display backend (creates Wayland window for LVGL) |
| libsdl2-mixer | Audio playback (WAV files via PulseAudio) |
| libgpiod / libgpiod-tools | GPIO sensor access (library + debug tools like gpioinfo) |
| goball-config | Device configuration (udev rules, WiFi, SSH keys, EEPROM) |

Display Stack:

| Package | Why |
| --- | --- |
| weston | Wayland compositor — manages the display |
| weston-init | systemd service and config for Weston |
| mesa-megadriver | OpenGL ES driver for the RPi5 GPU (V3D) |
| libegl-mesa / libgles2-mesa | EGL and OpenGL ES libraries (required by Weston) |
| libgbm | Generic Buffer Manager (GPU memory allocation) |

Audio Stack:

| Package | Why |
| --- | --- |
| pulseaudio / pulseaudio-server | Sound server (mixing, routing) |
| pulseaudio-module-alsa-sink/source | ALSA backend modules for PulseAudio |
| alsa-utils / alsa-plugins | Low-level audio tools and ALSA compatibility |

System Utilities:

| Package | Why |
| --- | --- |
| kernel-modules | All kernel modules (GPIO, PIO, audio, networking) |
| rpi-eeprom | RPi5 EEPROM firmware update tool |
| rpi-gpio | GPIO utility scripts |
| i2c-tools / devmem2 | Hardware debugging tools |
| nano / htop | Basic text editor and process monitor (dev convenience) |

Networking:

| Package | Why |
| --- | --- |
| networkmanager / networkmanager-nmcli | Network management (WiFi, Ethernet) |
| wpa-supplicant | WPA/WPA2 WiFi authentication |
| linux-firmware-rpidistro-bcm43455 | WiFi firmware for the RPi5’s Broadcom chip |
| iw | WiFi diagnostic tool |

### 5.4.2	Special Configuration

IMAGE_FEATURES += "ssh-server-openssh splash"

- ssh-server-openssh: Installs OpenSSH server for remote access

- splash: Enables the psplash boot splash screen

inherit core-image extrausers

- core-image: Base class for building root filesystem images

- extrausers: Allows setting user passwords at build time

disable_getty(): Creates a symlink from getty@tty1.service to /dev/null, effectively disabling the text login prompt on tty1. This is necessary because Weston takes over tty1 for display output.

## 5.5	recipes-goball/goball/goball_1.0.bb — The Application Recipe

This is the most important recipe — it builds the GoBall application from source.

SUMMARY = "GoBall Mini Golf Scoring System"
DESCRIPTION = "LVGL-based mini golf scoring application with \
    GPIO sensors and LED support"
LICENSE = "MIT"
LIC_FILES_CHKSUM = \
    "file://${COMMON_LICENSE_DIR}/MIT;md5=0835ade698e0bcf8506ecda2f7b4f302"

DEPENDS = "libsdl2 libsdl2-mixer libgpiod"

SRC_URI = "git://github.com/aabdelghani/GoBall.git;\
    protocol=https;branch=master \
           file://goball.service \
           file://pulseaudio-system.service"
SRCREV = "${AUTOREV}"

S = "${WORKDIR}/git"

inherit cmake systemd pkgconfig

EXTRA_OECMAKE = "-DCMAKE_BUILD_TYPE=Debug \
    -DYOCTO_BUILD=ON \
    -DSOUND_DIR_PATH=/opt/goball/sounds/"

SYSTEMD_SERVICE:${PN} = "goball.service \
    pulseaudio-system.service"
SYSTEMD_AUTO_ENABLE = "enable"

do_install() {
    install -d ${D}${bindir}
    install -m 0755 ${B}/SquareLine_Project ${D}${bindir}/goball

    install -d ${D}/opt/goball/sounds
    for f in $(find ${S}/modules/game_sounds -name '*.wav'); do
        install -m 0644 "$f" ${D}/opt/goball/sounds/
    done

    install -d ${D}${systemd_system_unitdir}
    install -m 0644 ${WORKDIR}/goball.service \
        ${D}${systemd_system_unitdir}/goball.service
    install -m 0644 ${WORKDIR}/pulseaudio-system.service \
        ${D}${systemd_system_unitdir}/pulseaudio-system.service
}

FILES:${PN} += "/opt/goball /opt/goball/sounds"

### 5.5.1	Line-by-Line Explanation

Metadata:

| Variable | Purpose |
| --- | --- |
| SUMMARY / DESCRIPTION | Human-readable package description |
| LICENSE = "MIT" | The project’s open-source license |
| LIC_FILES_CHKSUM | MD5 checksum of the license file (BitBake verifies this to ensure the license hasn’t changed) |

Dependencies:

DEPENDS = "libsdl2 libsdl2-mixer libgpiod"

Build-time dependencies. BitBake ensures these are compiled and their headers/libraries are available in the sysroot before building GoBall.

Source:

SRC_URI = "git://github.com/aabdelghani/GoBall.git;protocol=https;branch=master \
           file://goball.service \
           file://pulseaudio-system.service"
SRCREV = "${AUTOREV}"

- Fetches the GoBall source from GitHub (master branch)

- Also includes two local files from the files/ directory

- AUTOREV means “always use the latest commit” (good for development; pin to a specific SHA for production)

Source directory:

S = "${WORKDIR}/git"

After do_fetch and do_unpack, the Git clone is at ${WORKDIR}/git. This tells BitBake where to find the source code.

Build system:

inherit cmake systemd pkgconfig

- cmake: Use CMake for configure/compile (calls cmake and make)

- systemd: Automatically enables/disables systemd services

- pkgconfig: Sets up pkg-config for finding libraries

CMake flags:

EXTRA_OECMAKE = "-DCMAKE_BUILD_TYPE=Debug \
    -DYOCTO_BUILD=ON \
    -DSOUND_DIR_PATH=/opt/goball/sounds/"

| Flag | Effect in CMakeLists.txt |
| --- | --- |
| -DCMAKE_BUILD_TYPE=Debug | Debug build with level 4 logging |
| -DYOCTO_BUILD=ON | Skips local SDL2 paths and toolchain file (Yocto provides these) |
| -DSOUND_DIR_PATH=/opt/goball/sounds/ | Overrides the sound directory path (compiled into the binary) |

systemd integration:

SYSTEMD_SERVICE:${PN} = "goball.service pulseaudio-system.service"
SYSTEMD_AUTO_ENABLE = "enable"

Registers both services with systemd. enable means they start automatically on boot.

Custom install (do_install):

The do_install() function runs after compilation and places files into the staging directory (${D} = destination root):

- Binary: Copies SquareLine_Project binary to /usr/bin/goball

- Sounds: Copies all .wav files from the source tree to /opt/goball/sounds/

- Services: Installs both systemd service unit files

Package files:

FILES:${PN} += "/opt/goball /opt/goball/sounds"

Tells the packager to include /opt/goball/ in the goball package (by default, only standard paths like /usr/bin are included automatically).

### 5.5.2	goball.service — Application Service Unit

[Unit]
Description=GoBall Mini Golf Scoring System
After=weston.service sound.target pulseaudio-system.service
Wants=pulseaudio-system.service
Requires=weston.service

[Service]
Type=simple
Environment="SDL_VIDEODRIVER=wayland"
Environment="SDL_VIDEO_GL_DRIVER=libGLESv2.so.2"
Environment="SDL_AUDIODRIVER=pulseaudio"
Environment="WAYLAND_DISPLAY=wayland-1"
Environment="XDG_RUNTIME_DIR=/run/weston"
Environment="PULSE_SERVER=unix:/run/pulse/native"
ExecStartPre=/bin/sleep 2
ExecStart=/usr/bin/goball
Restart=on-failure
RestartSec=5
User=root
SupplementaryGroups=pulse-access

[Install]
WantedBy=multi-user.target

| Directive | Purpose |
| --- | --- |
| After=weston.service | Start after the display compositor is running |
| Requires=weston.service | Hard dependency — if Weston fails, GoBall doesn’t start |
| Wants=pulseaudio-system.service | Soft dependency — GoBall runs even if audio fails |
| SDL_VIDEODRIVER=wayland | Tell SDL2 to use Wayland (not X11 or fbdev) |
| SDL_AUDIODRIVER=pulseaudio | Tell SDL2 to use PulseAudio for audio output |
| WAYLAND_DISPLAY=wayland-1 | Connect to Weston’s Wayland socket |
| XDG_RUNTIME_DIR=/run/weston | Runtime directory where Weston creates its socket |
| PULSE_SERVER=unix:/run/pulse/native | PulseAudio socket path (system mode) |
| ExecStartPre=/bin/sleep 2 | Wait 2 seconds for Weston to fully initialize |
| Restart=on-failure | Auto-restart if the app crashes |
| SupplementaryGroups=pulse-access | Grant access to the PulseAudio socket |

### 5.5.3	pulseaudio-system.service — Audio Service

[Unit]
Description=PulseAudio System Mode
After=sound.target

[Service]
Type=notify
ExecStart=/usr/bin/pulseaudio --system --disallow-exit \
    --disallow-module-loading --log-target=journal
Restart=on-failure
RestartSec=3

[Install]
WantedBy=multi-user.target

PulseAudio runs in system mode (not per-user) because there’s no login session — Weston runs as root directly. The --disallow-exit flag prevents PulseAudio from shutting down when no clients are connected.

## 5.6	recipes-config/goball-config/goball-config_1.0.bb — Device Configuration

SUMMARY = "GoBall device configuration"
LICENSE = "MIT"
LIC_FILES_CHKSUM = \
    "file://${COMMON_LICENSE_DIR}/MIT;md5=0835ade698e0bcf8506ecda2f7b4f302"

SRC_URI = "file://99-pio.rules \
           file://Banhof.nmconnection \
           file://Ethernet.nmconnection \
           file://authorized_keys \
           file://rpi-eeprom-setup.sh \
           file://rpi-eeprom-setup.service"

inherit useradd systemd

SYSTEMD_SERVICE:${PN} = "rpi-eeprom-setup.service"
SYSTEMD_AUTO_ENABLE = "enable"

USERADD_PACKAGES = "${PN}"
GROUPADD_PARAM:${PN} = "-r pulse-access"

do_install() {
    # PIO udev rules
    install -d ${D}${sysconfdir}/udev/rules.d
    install -m 0644 ${WORKDIR}/99-pio.rules \
        ${D}${sysconfdir}/udev/rules.d/

    # WiFi auto-connect profile (NetworkManager)
    install -d ${D}${sysconfdir}/NetworkManager/system-connections
    install -m 0600 ${WORKDIR}/Banhof.nmconnection \
        ${D}${sysconfdir}/NetworkManager/system-connections/
    install -m 0600 ${WORKDIR}/Ethernet.nmconnection \
        ${D}${sysconfdir}/NetworkManager/system-connections/

    # SSH authorized keys for root
    install -d ${D}/root/.ssh
    install -m 0600 ${WORKDIR}/authorized_keys \
        ${D}/root/.ssh/authorized_keys
    chmod 700 ${D}/root/.ssh

    # EEPROM setup script + service
    install -d ${D}${bindir}
    install -m 0755 ${WORKDIR}/rpi-eeprom-setup.sh \
        ${D}${bindir}/rpi-eeprom-setup.sh
    install -d ${D}${systemd_system_unitdir}
    install -m 0644 ${WORKDIR}/rpi-eeprom-setup.service \
        ${D}${systemd_system_unitdir}/
}

RDEPENDS:${PN} += "rpi-eeprom"
FILES:${PN} += "/root/.ssh"

This recipe installs six configuration files:

### 5.6.1	99-pio.rules — PIO Device Permissions

SUBSYSTEM=="*-pio", GROUP="gpio", MODE="0660"

This udev rule gives the gpio group read/write access to PIO devices (/dev/pio0, etc.). Without this, only root can access PIO, and the LED controller initialization fails.

### 5.6.2	WiFi Profile (Banhof.nmconnection)

[connection]
id=Banhof
type=wifi
autoconnect=true
autoconnect-priority=100

[wifi]
ssid=Banhof
mode=infrastructure

[wifi-security]
key-mgmt=wpa-psk
psk=CHANGEME

[ipv4]
method=auto

NetworkManager auto-connects to this WiFi network on boot. The .sample file is versioned in Git; the actual file with the real password is .gitignored.

### 5.6.3	Ethernet Profile (Ethernet.nmconnection)

[connection]
id=Ethernet
type=ethernet
autoconnect=true
autoconnect-priority=200

[ipv4]
method=manual
addresses=10.0.0.2/24

Static IP (10.0.0.2) for direct laptop-to-Pi Ethernet connection. Priority 200 > WiFi’s 100, so Ethernet is preferred when both are connected.

### 5.6.4	EEPROM Setup Script (rpi-eeprom-setup.sh)

Runs once on first boot to configure the RPi5 EEPROM:

- Sets DISPLAY_DIAGNOSTIC=0 — hides the EEPROM bootloader diagnostic screen

- Sets NET_INSTALL_AT_POWER_ON=0 — disables the “install OS from network” prompt

- Creates a flag file (/var/lib/goball/eeprom-configured) to skip on subsequent boots

## 5.7	recipes-multimedia/ — Audio Libraries

### 5.7.1	libsdl2-mixer_2.8.1.bb — SDL2_mixer Recipe

SUMMARY = "SDL2 multi-channel audio mixer library"
LICENSE = "Zlib"
LIC_FILES_CHKSUM = \
    "file://LICENSE.txt;md5=fbb0010b2f7cf6e8a13bcac1ef4d2455"

DEPENDS = "libsdl2"

SRC_URI = "https://github.com/libsdl-org/SDL_mixer/releases/\
    download/release-${PV}/SDL2_mixer-${PV}.tar.gz"
SRC_URI[sha256sum] = "cb760211b056bfe44f4a1e180cc7cb201137e4d..."

S = "${WORKDIR}/SDL2_mixer-${PV}"

inherit cmake pkgconfig

EXTRA_OECMAKE = " \
    -DSDL2MIXER_WAVE=ON \
    -DSDL2MIXER_MP3=OFF \
    -DSDL2MIXER_FLAC=OFF \
    -DSDL2MIXER_MOD=OFF \
    -DSDL2MIXER_MIDI=OFF \
    -DSDL2MIXER_OPUS=OFF \
    -DSDL2MIXER_WAVPACK=OFF \
    -DSDL2MIXER_VORBIS=OFF \
"

PROVIDES = "libsdl2-mixer"
RPROVIDES:${PN} = "libsdl2-mixer"

Why a custom recipe? The default OE-Core recipe for SDL2_mixer may not exist or may pull in heavy dependencies (libvorbis, libflac, libmpg123). This recipe enables only WAV support — the only format GoBall uses — keeping the image small and dependency-free.

### 5.7.2	pulseaudio_%.bbappend — PulseAudio Fix

do_install:append() {
    sed -i 's/load-module module-native-protocol-unix.*/\
        load-module module-native-protocol-unix auth-anonymous=1/' \
        ${D}${sysconfdir}/pulse/system.pa
}

This modifies PulseAudio’s system.pa configuration to allow anonymous local connections. Without this, the GoBall process (running as root) cannot connect to PulseAudio’s UNIX socket in system mode.

## 5.8	recipes-graphics/ — Display Configuration

### 5.8.1	weston-init.bbappend — Kiosk Mode

FILESEXTRAPATHS:prepend := "${THISDIR}/${PN}:"
PACKAGECONFIG:append = " no-idle-timeout"

Disables Weston’s idle timeout so the screen never turns off.

### 5.8.2	weston.ini — Compositor Configuration

[core]
shell=kiosk-shell.so
require-input=false
idle-time=0

[output]
name=HDMI-A-1
mode=2560x720

[output]
name=HDMI-A-2
mode=2560x720

| Setting | Purpose |
| --- | --- |
| shell=kiosk-shell.so | Kiosk shell — single fullscreen app, no window decorations, no taskbar |
| require-input=false | Start even without keyboard/mouse connected |
| idle-time=0 | Never blank the screen |
| Output sections | Configure both HDMI ports to 2560x720 (whichever port is used) |

### 5.8.3	weston.service — Compositor Service

[Unit]
Description=Weston Wayland Compositor (Kiosk Mode)
After=systemd-user-sessions.service dbus.socket
ConditionPathExists=/dev/tty0

[Service]
Type=notify
ExecStartPre=/bin/sleep 5
ExecStartPre=/bin/sh -c 'mkdir -p /run/weston && chmod 0700 /run/weston'
Environment="XDG_RUNTIME_DIR=/run/weston"
ExecStart=/usr/bin/weston --modules=systemd-notify.so
User=root
TTYPath=/dev/tty1

Weston starts 5 seconds after boot (giving the GPU time to initialize), runs on tty1 as root, and creates its runtime directory at /run/weston.

## 5.9	recipes-core/psplash/ — Boot Splash

### 5.9.1	psplash_%.bbappend

FILESEXTRAPATHS:prepend := "${THISDIR}/files:"
SPLASH_IMAGES:raspberrypi5 = \
    "file://psplash-goball.png;outsuffix=default"
SRC_URI:append:raspberrypi5 = " file://framebuf.conf"

Replaces the default Poky splash screen with a custom GoBall-branded PNG (2560x720). The :raspberrypi5 override ensures this only applies when building for the RPi5 machine.

The framebuf.conf file is a systemd drop-in that fixes a framebuffer device dependency issue specific to the RPi5 (the device path differs from earlier models).

## 5.10	recipes-support/libgpiod/ — GPIO Library

### 5.10.1	libgpiod_1.6.5.bb

SUMMARY = "C library and tools for interacting with the \
    linux GPIO character device"
LICENSE = "LGPL-2.1-or-later"
LIC_FILES_CHKSUM = \
    "file://COPYING;md5=2caced0b25dfefd4c601d92bd15116de"

SRC_URI = "file://${BP}"
S = "${WORKDIR}/${BP}"

DEPENDS = "autoconf-archive-native"
inherit autotools pkgconfig

PACKAGECONFIG[tools] = "--enable-tools,--disable-tools"
PACKAGECONFIG = "tools"

PROVIDES = "libgpiod"
RPROVIDES:${PN} = "libgpiod"

PACKAGES =+ "${PN}-tools"
FILES:${PN}-tools = "${bindir}/*"

Why a custom recipe? GoBall requires libgpiod v1.6.x (the v1 API with gpiod_line_event_wait_bulk()). The OE-Core recipe provides v2.x which has an incompatible API. This recipe bundles the full libgpiod 1.6.5 source tree in files/libgpiod-1.6.5/ and builds it using autotools.

The ${BP} variable expands to libgpiod-1.6.5 (base package name + version), matching the local source directory.

## 5.11	recipes-core/dropbear/ — SSH Configuration

### 5.11.1	dropbear_%.bbappend

FILESEXTRAPATHS:prepend := "${THISDIR}/${PN}:"
PAM_SRC_URI = "file://dropbear"
PACKAGECONFIG:remove = "pam"

Disables PAM for the Dropbear SSH server. PAM on RPi5 has compatibility issues that prevent root login. The fallback SSH server is OpenSSH (added via IMAGE_FEATURES += "ssh-server-openssh"), so Dropbear is effectively unused but this append prevents build errors.

# 6	Kernel Configuration

## 6.1	Kernel Recipe

The kernel comes from meta-raspberrypi via the linux-raspberrypi recipe. It builds the official Raspberry Pi kernel with RPi5-specific patches and device tree overlays.

## 6.2	Required Kernel Features

| Feature | Config | Why |
| --- | --- | --- |
| GPIO character device | CONFIG_GPIO_CDEV | libgpiod accesses GPIOs via /dev/gpiochip0 |
| RP1 PIO module | CONFIG_RP1_PIO | WS2812 LED control via PIO hardware |
| V3D GPU driver | CONFIG_DRM_V3D | OpenGL ES for Weston compositor |
| KMS/DRM | CONFIG_DRM | Display output via kernel mode setting |
| ALSA | CONFIG_SND | Audio hardware access |
| USB host | CONFIG_USB | USB peripherals |
| WiFi (brcmfmac) | CONFIG_BRCMFMAC | RPi5 WiFi chip driver |

The PIO kernel module is explicitly included via local.conf:

MACHINE_EXTRA_RRECOMMENDS += "kernel-module-rp1-pio"

This ensures the rp1-pio module is packaged and loaded at boot, allowing GoBall’s piolib to open /dev/pio0 for LED control.

# 7	Building the Image

## 7.1	Quick Start (First Time)

# 1. Clone and set up everything
git clone https://github.com/aabdelghani/meta-goball.git
./meta-goball/setup-goball.sh ~/yocto/goball

# 2. Enter the build environment
cd ~/yocto/goball/poky
source oe-init-build-env build

# 3. Edit WiFi credentials
nano meta-goball/recipes-config/goball-config/files/Banhof.nmconnection

# 4. Add your SSH public key
cat ~/.ssh/id_ed25519.pub > \
    meta-goball/recipes-config/goball-config/files/authorized_keys

# 5. Build the image
bitbake goball-image

# 6. Flash to SD card
sudo bmaptool copy \
    tmp-glibc/deploy/images/raspberrypi5/\
    goball-image-raspberrypi5.rootfs.wic.bz2 \
    /dev/sdX

## 7.2	What setup-goball.sh Does

The setup script automates the initial environment:

- Clones Poky (Scarthgap branch) if not present

- Clones meta-raspberrypi and meta-openembedded layers

- Clones meta-goball from GitHub

- Sources oe-init-build-env to create the build/ directory

- Copies template configs (local.conf.sample, bblayers.conf.sample) to build/conf/

- Replaces ##POKYDIR## placeholder with the actual Poky path

- Creates .sample copies of sensitive files (WiFi credentials, SSH keys)

## 7.3	Rebuilding After Code Changes

When you push code changes to the GoBall GitHub repo:

cd ~/yocto/goball/poky
source oe-init-build-env build

# Clean the old build and rebuild
bitbake goball -c cleansstate
bitbake goball-image

cleansstate removes all cached build artifacts for the goball recipe, forcing a fresh clone and rebuild. Other packages (SDL2, kernel, etc.) remain cached.

## 7.4	Useful BitBake Commands

| Command | Purpose |
| --- | --- |
| bitbake goball-image | Build the full image |
| bitbake goball | Build only the GoBall package |
| bitbake goball -c cleansstate | Clean GoBall build cache |
| bitbake goball -c devshell | Open a shell in the build environment |
| bitbake -e goball \¦ grep ^S= | Show the source directory path |
| bitbake -g goball-image | Generate dependency graph |
| bitbake -s \¦ grep sdl | Search available recipes |

# 8	Deploying to Raspberry Pi 5

## 8.1	Flashing the SD Card

# Find your SD card device
lsblk

# Flash (bmaptool is faster than dd)
sudo bmaptool copy \
    tmp-glibc/deploy/images/raspberrypi5/\
    goball-image-raspberrypi5.rootfs.wic.bz2 \
    /dev/sdX

# Or with dd (slower)
bzcat tmp-glibc/deploy/images/raspberrypi5/\
    goball-image-raspberrypi5.rootfs.wic.bz2 \
    | sudo dd of=/dev/sdX bs=4M status=progress

## 8.2	First Boot

- Insert the SD card into the RPi5

- Connect HDMI display, sensors, and power

- The boot sequence is:

  - RPi5 EEPROM bootloader (hidden after first EEPROM config)

  - Custom psplash boot screen (GoBall branding)

  - Weston compositor starts (kiosk shell)

  - PulseAudio starts (system mode)

  - GoBall starts automatically

## 8.3	Updating Just the Application

To update GoBall without reflashing:

# Cross-compile on host
cd /home/q/1Projects/GoBall/SquareLine_Project
cmake -B build -DCMAKE_EXPORT_COMPILE_COMMANDS=ON
make -C build -j$(nproc)

# Copy to Pi
scp build/SquareLine_Project root@10.0.0.2:/usr/bin/goball

# Restart on Pi
ssh root@10.0.0.2 systemctl restart goball

## 8.4	Boot Service Chain

power on
  --> EEPROM bootloader (RPi5 firmware)
  --> kernel (direct boot, no U-Boot)
  --> systemd (PID 1)
    --> psplash (boot splash)
    --> rpi-eeprom-setup (one-shot, first boot only)
    --> pulseaudio-system.service (audio server)
    --> weston.service (display compositor)
      --> 5s delay (GPU init)
      --> goball.service (the application)
        --> 2s delay (wait for Wayland socket)
        --> /usr/bin/goball

# 9	Development Workflow

## 9.1	The YOCTO_BUILD CMake Flag

When building inside Yocto, the recipe passes -DYOCTO_BUILD=ON. In CMakeLists.txt:

if(NOT YOCTO_BUILD)
    set(CMAKE_TOOLCHAIN_FILE toolchain-aarch64.cmake)
endif()

if(NOT YOCTO_BUILD)
    set(SDL_FOLDER sdl2-dev-rpi64)
    include_directories(${PROJECT_SOURCE_DIR}/${SDL_FOLDER}/include)
    target_link_directories(${PROJECT_NAME} PRIVATE
        ${PROJECT_SOURCE_DIR}/${SDL_FOLDER}/lib
        ${PROJECT_SOURCE_DIR}/rpi5-sysroot/usr/lib/aarch64-linux-gnu)
endif()

This flag:

- Skips the local toolchain-aarch64.cmake (Yocto provides its own cross-compiler)

- Skips the bundled SDL2 headers/libraries (Yocto provides these via sysroot)

- Allows the same CMakeLists.txt to work for both Yocto builds and local cross-compilation

## 9.2	Adding a New Package to the Image

- Find the recipe: bitbake -s | grep <package>

- Add to goball-image.bb: IMAGE_INSTALL += "package-name"

- Rebuild: bitbake goball-image

If no recipe exists, create one in meta-goball/recipes-<category>/<name>/<name>_<version>.bb.

## 9.3	Modifying Kernel Configuration

To add a kernel config option:

# 1. Start a kernel devshell
bitbake virtual/kernel -c devshell

# 2. Run menuconfig
make menuconfig

# 3. Save and exit

# 4. Copy the changed config
# Create a .cfg fragment in meta-goball with just the changed options

Or create a .cfg fragment file and add it via a linux-raspberrypi_%.bbappend.

# 10	Troubleshooting

## 10.1	Common Build Errors

| Error | Cause | Fix |
| --- | --- | --- |
| Nothing PROVIDES 'libsdl2-mixer' | Missing recipe | Verify meta-goball is in bblayers.conf |
| do_fetch failed | Network issue or wrong SRCREV | Check internet; use SRCREV = "${AUTOREV}" for dev |
| QA Issue: No GNU_HASH | Missing compiler flags | Add -Wl,--hash-style=gnu to LDFLAGS |
| sstate cache miss | First build or different machine | Normal — takes longer on first build |
| Disk space | /tmp or build dir full | Set DL_DIR and SSTATE_DIR to larger disk |

## 10.2	Runtime Issues

| Symptom | Cause | Fix |
| --- | --- | --- |
| Black screen | U-Boot enabled | Set RPI_USE_U_BOOT = "0" in local.conf |
| No sound | PulseAudio auth | Check pulseaudio_%.bbappend has auth-anonymous=1 |
| No WiFi | Missing firmware | Verify linux-firmware-rpidistro-bcm43455 in IMAGE_INSTALL |
| No LEDs | PIO permissions | Check 99-pio.rules installed, kernel-module-rp1-pio loaded |
| App crashes | Missing Wayland socket | Check weston.service runs before goball.service |

## 10.3	Debugging Commands (on target)

# Check service status
systemctl status goball
systemctl status weston
systemctl status pulseaudio-system

# View application logs
journalctl -u goball -f

# Check GPIO
gpioinfo

# Check PIO
ls -la /dev/pio*

# Check audio
pactl info
aplay -l

# Check display
weston-info

# Rebuild and deploy quickly
# (on host)
bitbake goball -c cleansstate && bitbake goball
scp build/tmp-glibc/work/*/goball/*/image/usr/bin/goball \
    root@10.0.0.2:/usr/bin/goball
ssh root@10.0.0.2 systemctl restart goball

# 11	Appendix A: Deployment Configuration Reference

The file meta-goball/deploy-config.conf contains a complete reference of every parameter that must be reviewed when deploying to a new machine. Parameters are marked as:

- [REBUILD] — Requires a Yocto rebuild and SD card reflash

- [CODE] — Requires a code change, recipe clean, and rebuild

- [RUNTIME] — Can be changed on the running device via SSH

Key deployment parameters:

| Parameter | Default | Type | File |
| --- | --- | --- | --- |
| WiFi SSID/Password | Banhof/CHANGEME | REBUILD | Banhof.nmconnection |
| Ethernet IP | 10.0.0.2/24 | REBUILD | Ethernet.nmconnection |
| HDMI Port | HDMI-A-2 | REBUILD | local.conf, weston.ini |
| Display Resolution | 2560x720 | REBUILD+CODE | local.conf, weston.ini, main.c |
| GPIO Sensor Pins | 17,26,27,24 | CODE | gpio_event.h |
| LED Strip GPIOs | 3, 2 | CODE | main.c |
| LED Pixel Count | 144 | CODE | led_logic_event.h |
| Sound Directory | /opt/goball/sounds/ | REBUILD | goball_1.0.bb |
| Root Password | 123 | REBUILD | goball-image.bb |
| SSH Keys | (your key) | REBUILD | authorized_keys |
| Build Type | Debug | REBUILD | goball_1.0.bb |

# 12	Appendix B: Yocto/Poky Version Information

| Component | Version | Branch |
| --- | --- | --- |
| Poky | 5.0 | Scarthgap |
| BitBake | 2.8 | Scarthgap |
| GCC (cross) | 14.x | Provided by OE-Core |
| Linux kernel | 6.6.x | linux-raspberrypi |
| meta-raspberrypi | Scarthgap | Scarthgap |
| meta-openembedded | Scarthgap | Scarthgap |